A/B Updates and Read-Only Root

OTA update strategies and filesystem hardening for embedded products

Over-the-air (OTA) firmware updates are essential for shipping embedded products: security patches, bug fixes, feature additions. The A/B update strategy is the industry standard for safe OTA — it keeps the device running on a known-good image while writing the new one, then atomically switches over. This guide covers A/B design, popular OTA frameworks (SWUpdate, Mender, RAUC), and the read-only root filesystem that makes all of it safe.

How to read Parts 1-2 cover the why and design of A/B updates. Part 3 covers SWUpdate. Part 4 covers Mender. Part 5 covers RAUC. Part 6 covers read-only rootfs and dm-verity. Use Parts 1-2 to decide your approach, then go deep on whichever framework fits your product.

What you should be able to do after reading

- Design an A/B partition layout for your product

- Choose between SWUpdate, Mender, and RAUC

- Configure a read-only rootfs with persistent data partition

- Understand how dm-verity provides cryptographic rootfs integrity

- Integrate SWUpdate or RAUC into a Yocto build

Part 1 — Why A/B Updates

Learning objectives - Explain the risks of in-place updates - Describe the A/B model and its guarantees

1.1 The Problem with In-Place Updates

The naive update approach: download new firmware, overwrite the running system. Problems:

- Power loss during write — system is half-updated, unbootable. Bricked device in the field.

- Failed update — new version has a bug, or installation fails partway. Same brick problem.

- No rollback — if the new version works but causes issues days later, rolling back means re-flashing with the old firmware (which you might not have).

1.2 The A/B Model

Two complete copies of the system partition exist (A and B). One is active; the other is standby.

Normal operation: [BOOT] [SYSTEM_A (active, running)] [SYSTEM_B (standby)] [DATA] OTA update arrives: 1. Write new firmware to SYSTEM_B (device keeps running from SYSTEM_A) 2. Verify the write (checksum, signature) 3. Set bootloader flag: 'try SYSTEM_B next boot' 4. Reboot 5. Bootloader boots SYSTEM_B 6. New firmware runs — if it boots successfully and reports 'OK': → Mark SYSTEM_B as permanently active 7. If it fails to boot or report OK within timeout: → Bootloader reverts to SYSTEM_A → Device back to known-good state, sends failure report

1.3 Key Properties

- Atomicity — the switch from old to new is a single bootloader flag change. Either A or B; never half.

- Automatic rollback — if the new firmware fails to confirm itself, revert is automatic.

- Zero downtime — new firmware is written while the device is fully operational.

- Testability — new firmware runs on real hardware before being committed as permanent.

Part 2 — Partition Design

2.1 The Standard A/B Layout

Partition Label Size Filesystem Purpose ───────── ───── ──── ────────── ─────────── mmcblk0p1 BOOT 64 MB FAT32 Bootloader, kernel, DTB mmcblk0p2 SYSTEM_A 512 MB ext4 (RO) Active rootfs mmcblk0p3 SYSTEM_B 512 MB ext4 (RO) Standby rootfs for OTA mmcblk0p4 DATA 512 MB ext4 (RW) Persistent data, logs, config

2.2 What Goes Where

- BOOT — SPL/MLO, U-Boot, kernel, DTB, extlinux.conf or bootloader scripts

- SYSTEM_A / SYSTEM_B — complete rootfs. Mounted read-only. Identical layout; different content between versions.

- DATA — everything that needs to persist across updates: /etc/ device-specific config, /var/log/, application state, provisioning data

DATA partition design for Allegion For Allegion products: plan the DATA partition layout before shipping. It's the only partition that survives a firmware update. Typical layout: - /data/config/ — device-specific settings (network, credentials, provisioning) - /data/log/ — persistent log files (bind-mounted to /var/log) - /data/state/ — application state that persists across reboots and updates - /data/update/ — staging area for OTA packages Use symlinks or bind mounts to make the rootfs see /var/log as /data/log. Documented in systemd-volatile-root or a custom mount unit.

Part 3 — SWUpdate

Learning objectives - Understand SWUpdate's architecture - Create a .swu update package - Integrate with Yocto

3.1 What SWUpdate Is

SWUpdate is an open-source Linux update framework from Stefano Babic / Denx. It runs as a daemon on the device, accepts update packages (.swu files), verifies signatures, and applies updates using a handler pipeline. Extremely configurable; widely used in industrial embedded Linux.

3.2 SWUpdate Architecture

- swupdate daemon — runs on the device, accepts update packages

- .swu file — a CPIO archive containing a sw-description (manifest) and the actual update payloads

- Handlers — pluggable; handle raw flash writes, ext4 images, shell scripts, encrypted images, boot env updates

- Mongoose web server — optional; provides a web UI for update upload

- Suricatta — optional; HAWKBIT cloud backend integration for fleet management

3.3 A Simple sw-description

software = { version = "1.2.3"; description = "Allegion controller update"; hardware-compatibility: ["1.0", "1.1"]; images: ( { filename = "system-image.ext4"; type = "raw"; device = "/dev/mmcblk0p3"; /* write to SYSTEM_B */ sha256 = "abc123..."; encrypted = true; } ); scripts: ( { filename = "post-install.sh"; type = "shellscript"; sha256 = "def456..."; } ); bootenv: ( { name = "boot_part"; value = "2"; /* tell bootloader to try SYSTEM_B */ } ); }

3.4 Yocto Integration

# Add meta-swupdate layer $ git clone https://github.com/sbabic/meta-swupdate $ bitbake-layers add-layer meta-swupdate # In your image recipe: IMAGE_INSTALL += "swupdate" # Build an .swu file via Yocto # (meta-swupdate provides swupdate-image class)

Part 4 — Mender

4.1 What Mender Is

Mender is an end-to-end OTA update solution: client daemon on device, server (Mender Hosted or self-hosted), and management dashboard. It handles A/B partition switching automatically using U-Boot integration.

- Strengths: polished UX, hosted cloud option, good documentation, active community

- Trade-offs: requires specific partition layout, U-Boot integration, Mender server. More opinionated than SWUpdate.

4.2 How Mender Works

- Device runs mender client daemon

- Client polls Mender server for pending updates

- Server delivers signed .mender artifact

- Client writes to inactive partition

- Client sets U-Boot upgrade_available=1 env var

- Device reboots into new partition

- Client verifies new firmware works, calls mender commit

- If not committed within timeout: U-Boot rolls back automatically

Part 5 — RAUC

5.1 What RAUC Is

RAUC (Robust Auto-Update Controller) is a lightweight, security-focused update framework. Like SWUpdate but with a stronger focus on security (mandatory signing), systemd integration, and D-Bus interface.

- Mandatory code signing — no unsigned updates accepted

- systemd integration — RAUC service, D-Bus interface for scripting

- Multiple backend support — works with Barebox, U-Boot, GRUB

- Hawkbit integration — connects to Hawkbit update server for fleet management

5.2 RAUC Bundle

# RAUC manifest file (manifest.raucm) [update] compatible=Allegion-Controller-1 version=1.2.3 [image.rootfs] filename=rootfs.ext4 sha256=abc123... # Build and sign a bundle $ rauc bundle --cert=allegion.crt --key=allegion.key \ manifest.raucm rootfs.ext4 update.raucb # Install on device $ rauc install update.raucb

Part 6 — Read-Only Root and dm-verity

Learning objectives - Enable read-only rootfs in Yocto - Configure dm-verity for cryptographic rootfs integrity

6.1 Read-Only Rootfs in Yocto

# In your image recipe: IMAGE_FEATURES += "read-only-rootfs" # This sets the kernel command line to 'ro' # and configures systemd to handle the resulting # read-only filesystem gracefully # Verify on target: $ mount ¦ grep ' / ' /dev/mmcblk0p2 on / type ext4 (ro,relatime)

6.2 dm-verity — Cryptographic Rootfs Integrity

dm-verity is a kernel feature that builds a hash tree over a block device. Any modification to the device (attacker writes to rootfs) is detected instantly. Used by Android, Chrome OS, and production embedded Linux.

# Generate verity metadata $ veritysetup format rootfs.ext4 rootfs.verity Root hash: abcdef1234... Salt: ... # The root hash is baked into the signed bootloader (U-Boot FIT image) # or kernel command line (also signed) # At boot, kernel activates dm-verity: # dm-verity=1,/dev/mmcblk0p2,/dev/mmcblk0p3,<hash>,<salt> # Any bit flip in the rootfs = immediate kernel panic # (or remount read-only, depending on on_error config)

Common pitfalls - Forgetting to update DATA partition bindings after a rootfs update. If your new rootfs moves a file path, the bind mounts from DATA to rootfs break silently. Test update scenarios before release. - Not committing the Mender update. If mender-commit isn't called, the device reboots back to the old firmware after the timeout. Your application must call it after confirming it's healthy. - Writing to SYSTEM_A/B during development. If developers have a writable rootfs and accidentally write to it, the checksums for dm-verity will be wrong. Use read-only rootfs from day one. - SWUpdate handler not restoring bootloader env. If your update script crashes before setting the bootloader flag, the device boots the new system on next reboot without the rollback protection being armed. - Same size for SYSTEM_A and SYSTEM_B. They must be the same size. If your image ever grows larger than the partition, the update will fail.

Try this yourself - Enable IMAGE_FEATURES += "read-only-rootfs" in a test image. Boot it. Try touch /test. Observe the error. - Look at SWUpdate's sw-description format. Write a sw-description for a hypothetical update that writes a rootfs image and changes a bootloader env var. - Set up SWUpdate or RAUC in a Yocto build. Generate an update package. Use swupdate to apply it to a test VM. - Research dm-verity. Run veritysetup format on a small ext4 image. Note the root hash output.

References

- SWUpdate documentation: sbabic.github.io/swupdate/

- Mender documentation: docs.mender.io

- RAUC documentation: rauc.readthedocs.io

- dm-verity kernel docs: kernel.org/doc/html/latest/admin-guide/device-mapper/verity.html

- meta-swupdate: github.com/sbabic/meta-swupdate

- meta-mender: github.com/mendersoftware/meta-mender

- meta-rauc: github.com/rauc/meta-rauc