Boot, Flash, and Verify on Target

Day 4 · Session 1 — getting your image running on real hardware

This session covers the moment your Yocto build meets real hardware (or QEMU): understanding the boot flow, choosing the right flashing method for your artifact type, and running a disciplined first-boot checklist. Each step is explained with what it does, why it matters, and what breaks if you skip it.

How to read this session Part 1 is the boot flow (what happens from power-on). Part 2 is flashing methods by artifact type. Part 3 is the first-boot checklist. Cross-references point to the deeper standalone guides where a topic has its own document.

Session goals

- Trace the boot flow from power-on to running services

- Pick the correct flashing method for wic, sdcard, or raw-partition artifacts

- Run a first-boot checklist that catches problems before they reach the field

Part 1 :The Boot Flow

Learning objectives - Name each stage from power-on to systemd - Know where to look when boot stops at each stage

1.1 The Four Stages

Every embedded Linux boot follows the same chain. When something fails, your first diagnostic question is always: which stage did it stop at?

Power on │ ▼ Bootloader (ROM → SPL → U-Boot, or RPi GPU firmware) • initializes DRAM, clocks, storage • loads kernel + device tree into memory • passes bootargs (kernel command line) │ ▼ Kernel (zImage / Image / kernel7.img) • decompresses, initializes subsystems and drivers • mounts the root filesystem • starts PID 1 (init) │ ▼ init/systemd (PID 1) • mounts remaining filesystems (/var, /data) • brings up targets in dependency order │ ▼ Services (your application, networking, SSH, logging) • systemd starts each unit per its dependencies • system reaches multi-user.target (operational)

Cross-reference: the bootloader stage is covered in depth in the Bootloader and U-Boot guide; the device tree the bootloader passes is in the Device Tree and Debugging guide.

1.2 Where Boot Stops, and What It Means

- Nothing on serial console at all — bootloader isn't running. Wrong/corrupt SPL, bad DRAM init, wrong boot media, or serial cable miswired (RX/TX swapped).

- Bootloader prints, then silence — kernel didn't load or didn't start. Wrong kernel address, missing DTB, wrong bootargs, or kernel image corrupt.

- Kernel prints, then "Kernel panic - not syncing: VFS: Unable to mount root fs" — rootfs not found. Wrong root= in bootargs, missing filesystem driver, or unflashed/corrupt rootfs partition.

- Kernel boots, then hangs before login — systemd stuck on a service. A unit is failing or waiting on a dependency (often network or a mount).

- Login appears but a service is missing — the service failed to start. systemctl status <service> and journalctl -u <service> tell you why.

Part 2: Flashing Methods by Artifact Type

Learning objectives - Match the flashing method to the artifact - Flash safely without destroying the wrong disk

2.1 Identify the Target Device First - Every Time

lsblk — identify the SD card / eMMC What it does: Lists block devices with sizes and mount points so you can identify which device is your SD card (e.g. /dev/sdb, 8G). Why it matters: Flashing writes to a raw device. Picking the wrong one destroys whatever is on it. The size and removable flag tell you which device is the card. If you skip it: You risk running dd/bmaptool against your workstation's main disk (/dev/sda) and wiping your operating system. This is the single most destructive mistake in embedded work.

$ lsblk NAME SIZE TYPE MOUNTPOINT sda 476G disk ← your workstation disk — NEVER target this ├─sda1 512M part /boot/efi └─sda2 475G part / sdb 7.4G disk ← the SD card (matches its labelled size) └─sdb1 7.4G part /media/user/... # Unmount any auto-mounted partitions before flashing $ sudo umount /dev/sdb* 2>/dev/null

2.2 Artifact Type → Flashing Method

Artifact Method Notes ──────── ────── ───── .wic / .wic.bz2 + .wic.bmap bmaptool fastest, verified, modern .wic dd works, writes every block .rpi-sdimg dd RPi full-card image .sdcard (TI boards) dd full-card image w/ partitions .tar.bz2 (rootfs only) tar -x extract onto a pre-formatted part raw partitions (separate) dd per part boot + rootfs flashed separately

2.3 Method A:  bmaptool (recommended for wic)

bmaptool copy What it does: Writes a .wic/.wic.bz2 image to the card, using the .bmap file to skip empty blocks and verify checksums as it goes. Why it matters: It only writes blocks that actually contain data. A 4 GB image with 200 MB of real content writes in ~30s instead of ~8 min, and verifies integrity automatically. If you skip it: Without bmaptool you fall back to dd, which writes all 4 GB including empty space — 5-10x slower and with no built-in verification.

$ cd tmp/deploy/images/raspberrypi3/ $ sudo bmaptool copy core-image-minimal-raspberrypi3.wic.bz2 /dev/sdb # bmaptool auto-finds the matching .wic.bmap file # Requires wic, wic.bz2, wic.bmap all in IMAGE_FSTYPES

Cross-reference: full bmaptool vs dd vs rpi-sdimg discussion is in the Image Engineering guide and the earlier Q&A on wic generation.

2.4 Method B : dd (universal fallback)

dd What it does: Copies an image byte-for-byte to the raw block device. Why it matters: It works for any full-disk image (.wic, .rpi-sdimg, .sdcard) and needs no extra tooling. The bs=4M makes it write in efficient chunks. If you skip it: If you omit status=progress you get no feedback (looks frozen). If you omit the final sync, data stays buffered and the card may be incomplete when you pull it out.

$ sudo dd if=core-image-minimal-raspberrypi3.rpi-sdimg \ of=/dev/sdb bs=4M status=progress $ sync # CRITICAL: flush buffers before removing the card

2.5 Method C : tar (rootfs onto a pre-formatted partition)

Used when boot and rootfs are flashed separately (common on BeagleBone). The boot files go on a FAT partition; the rootfs tarball is extracted onto an ext4 partition.

tar -x onto the rootfs partition What it does: Extracts the rootfs tarball onto an already-formatted, mounted ext4 partition. Why it matters: Lets you update just the rootfs without re-partitioning, and preserves the partition table and bootloader already on the card. If you skip it: If the partition isn't formatted/mounted first, files land in the wrong place. If you forget to extract as root (sudo), file ownership and permissions are wrong and the system may not boot.

$ sudo mount /dev/sdb2 /mnt/root $ sudo tar -xf core-image-minimal-beaglebone-yocto.tar.bz2 -C /mnt/root/ $ sync && sudo umount /mnt/root

Part 3: The First-Boot Checklist

Learning objectives - Verify the six things that prove a healthy boot - Catch field problems before deployment

After flashing and booting, run these checks in order. Each one verifies a subsystem; together they prove the device is healthy.

3.1 Clocks and Time

date / timedatectl What it does: Shows the system clock and whether time sync (NTP/chrony) is active. Why it matters: Many failures cascade from wrong time: TLS certificate validation fails, logs have wrong timestamps, scheduled tasks misfire. Security products especially depend on correct time. If you skip it: If time is wrong and you don't notice, TLS to the cloud fails with confusing 'certificate not yet valid' errors and log forensics become unreliable.

$ date $ timedatectl # Check: System clock synchronized: yes # NTP service: active

3.2 Networking

ip addr / ip route / ping What it does: Shows interface addresses, the default route, and basic reachability. Why it matters: Confirms the device got an IP, knows its gateway, and can reach the network. This is the diagnostic ladder from the Network/SSH guide. If you skip it: Skipping this means discovering connectivity problems later in the field, where debugging is far harder than on the bench.

$ ip addr show # do I have an IP? $ ip route show default # do I have a gateway? $ ping -c3 8.8.8.8 # can I reach the internet? $ getent hosts example.com # does DNS resolve?

Cross-reference: full diagnostic ladder and field networking in the Network Essentials and SSH Hygiene guide.

3.3 Storage Mounts

mount / df -h / lsblk What it does: Shows which filesystems are mounted, where, and how full they are. Why it matters: Confirms the rootfs mounted correctly (ro or rw as intended) and that data/log partitions are present. On read-only-rootfs designs, verifies the writable partitions are mounted. If you skip it: If /data or /var isn't mounted, the application writes to a tmpfs or read-only location — data silently lost on reboot, or writes fail.

$ mount ¦ grep ' / ' # rootfs: ro or rw as intended? $ df -h # space usage per filesystem $ lsblk # all partitions present and mounted?

3.4 Logs

journalctl -b / dmesg What it does: Shows the current boot's log (journalctl -b) and kernel ring buffer (dmesg). Why it matters: Reveals errors and warnings from boot: failed drivers, missing firmware, service start failures. The fastest way to find what went wrong. If you skip it: Skipping the log review means shipping a device with silent errors (a failed-but-restarted service, a missing firmware warning) that bite you later.

$ journalctl -b -p err # errors from this boot only $ dmesg ¦ grep -i 'fail\¦error\¦warn' ¦ head $ systemctl --failed # any units that failed to start?

Cross-reference: journalctl patterns and persistent logs are covered in detail in the Day-to-Day Operations session (next).

3.5 CPU and Memory Sanity

free -h / top / uptime What it does: Shows memory usage, running processes, load average. Why it matters: Confirms the device isn't memory-starved or CPU-pegged at idle. A device that's at 90% memory at boot has no headroom for real work. If you skip it: Skipping this hides a memory leak or runaway process until the device crashes in the field under load.

$ free -h # memory: is there headroom? $ uptime # load average — should be low at idle $ top -bn1 ¦ head -15 # any process eating CPU at idle?

Cross-reference: full performance methodology in the Performance Checks session.

3.6 The Checklist as a Script

Worked example: a first-boot health script #!/bin/sh # first-boot-check.sh — run on target after flashing echo '=== TIME ==='; date; timedatectl ¦ grep -i synchron echo '=== NETWORK ==='; ip -br addr; ip route show default ping -c2 -W2 8.8.8.8 >/dev/null && echo 'net: OK' ¦¦ echo 'net: FAIL' echo '=== STORAGE ==='; df -h ¦ grep -v tmpfs echo '=== FAILED UNITS ==='; systemctl --failed --no-legend echo '=== ERRORS ==='; journalctl -b -p err --no-pager ¦ tail -10 echo '=== RESOURCES ==='; free -h; uptime. Ship this script in your dev image. Running it after every flash turns the checklist into one command and makes 'is this device healthy?' answerable in seconds.

Common pitfalls - Pulling the card before sync completes. dd returns before the kernel finishes writing. Always sync and wait. Removing early = corrupt card, mysterious boot failures. - Flashing the wrong device. Triple-check lsblk. /dev/sda is almost always your workstation disk; /dev/sdb or /dev/mmcblk is usually the card. - Skipping the log check because 'it booted to login'. Booting to login doesn't mean healthy. A failed service can leave the device up but non-functional. Always run systemctl --failed. - Ignoring wrong time. A device with a dead RTC battery boots to 1970. TLS breaks, logs are useless. Catch it in the time check, not in the field. - Not testing on real hardware. QEMU boots don't exercise the real bootloader, real storage timing, or real peripherals. Always do a hardware first-boot before release.

Try this yourself - Flash an image to a card with bmaptool. Time it. Flash the same image with dd. Compare the times. - Boot a board. Deliberately corrupt bootargs (wrong root=) at the U-Boot prompt. Observe the VFS kernel panic. Fix it. - Write the first-boot-check.sh script. Run it on a freshly flashed device. Confirm all six sections report healthy. - Run systemctl --failed on a working device. If empty, disable a service's dependency to make one fail, then observe how it shows up.

References

- Bootloader and U-Boot study guide (this series) — boot stages in depth

- Device Tree and Debugging study guide — what the bootloader passes to the kernel

- Network Essentials and SSH Hygiene study guide — the diagnostic ladder

- Image Engineering study guide — artifact types and WIC

- bmap-tools: github.com/intel/bmap-tools