Day-to-Day Operations

Day 4 · Session 2 — running and maintaining a device in the field

Once a device is deployed, the work shifts from building to operating: managing user access, controlling services with systemd, reading logs to troubleshoot, handling field networking, and keeping storage reliable. This session covers each with the what/why/if-skipped structure.

How to read this session Five parts: user access and SSH, systemd service management, logging and troubleshooting, field networking, storage reliability. SSH hardening and full networking have dedicated guides; here we cover the operational day-to-day and cross-reference for depth.

Session goals

- Manage users and SSH access safely on a deployed device

- Control services with systemd (enable, disable, override, targets)

- Troubleshoot using journalctl patterns and persistent logs

- Handle field networking: DHCP vs static, DNS, firewall

- Apply storage reliability basics: fs checks, wear, partition strategy

Part 1: User Access and SSH

Learning objectives - Manage users and keys on a deployed device - Apply SSH hardening basics

1.1 User Management Essentials

useradd / passwd / usermod What it does: Creates users, sets passwords, and manages group membership on the device. Why it matters: Production devices should not run everything as root. A dedicated service user limits the blast radius if the application is compromised. If you skip it: Running the app as root means any application bug becomes a full system compromise. A compromised service user can only damage what that user can access.

# Create a non-login service user for your app $ useradd -r -s /usr/sbin/nologin appsvc # Add a field-service user to the sudo/wheel group $ useradd -m -G wheel fieldsvc $ passwd fieldsvc # In Yocto, do this declaratively in a recipe with useradd class # (see Recipes In Depth) rather than manually post-deploy

1.2 SSH Key Management

ssh-keygen / authorized_keys What it does: Generates key pairs and installs public keys for passwordless, key-based authentication. Why it matters: Keys are far stronger than passwords — no brute force, no phishing, no reuse. Key-based auth is the baseline for any remote access to a deployed device. If you skip it: Password SSH on a deployed device is a standing invitation to brute-force bots. Within hours of being internet-reachable, the logs fill with login attempts.

# Generate an Ed25519 key (on the operator's machine) $ ssh-keygen -t ed25519 -f ~/.ssh/field_key # Install the public key on the device $ ssh-copy-id -i ~/.ssh/field_key.pub fieldsvc@device # Then disable password auth in /etc/ssh/sshd_config: # PasswordAuthentication no # PermitRootLogin no

Cross-reference: the Network Essentials and SSH Hygiene guide covers the full hardened sshd_config, on-demand SSH, host key distribution, and fleet key management in depth. Treat this section as the operational summary; that guide is the reference.

Part 2: Service Management with systemd

Learning objectives - Enable, disable, start, stop services - Use override files instead of editing units - Understand targets

2.1 The Core Commands

systemctl enable/disable/start/stop What it does: enable/disable controls whether a service starts at boot; start/stop controls it right now. Why it matters: enable and start are different things. enable sets boot behavior (persistent); start affects only the current session. Confusing them is the most common systemd mistake. If you skip it: If you only start a service but forget to enable it, it works until the next reboot, then silently doesn't come up. If you only enable without start, it won't run until reboot.

$ systemctl status myapp # is it running? recent logs $ systemctl start myapp # start now (this session) $ systemctl enable myapp # start at every boot (persistent) $ systemctl enable --now myapp # both at once $ systemctl disable myapp # don't start at boot $ systemctl restart myapp # stop then start $ systemctl reload myapp # re-read config without full restart

2.2 Override Files — Never Edit Shipped Units

systemctl edit (drop-in override) What it does: Creates a drop-in override file in /etc/systemd/system/<unit>.d/ that overlays the shipped unit without modifying it. Why it matters: The shipped unit in /lib/systemd/system/ is replaced on every firmware update. Overrides in /etc/ survive updates and keep your local changes separate from vendor defaults. If you skip it: If you edit the shipped unit directly, your change is wiped out on the next OTA update and the service silently reverts to default behavior — a confusing field regression.

# Create an override (opens an editor) $ systemctl edit myapp # This creates /etc/systemd/system/myapp.service.d/override.conf # Example override: restart on failure, add a resource limit [Service] Restart=on-failure RestartSec=5 MemoryMax=100M # Apply $ systemctl daemon-reload $ systemctl restart myapp

2.3 Targets

Targets are systemd's grouping mechanism (the rough equivalent of old SysV runlevels):

- multi-user.target — normal operational state (networking, services, no GUI). Most embedded devices boot here.

- graphical.target — multi-user + display manager.

- rescue.target — single-user, minimal services, for recovery.

- emergency.target — bare minimum, read-only root, for serious recovery.

$ systemctl get-default # what target boots by default $ systemctl set-default multi-user.target $ systemctl isolate rescue.target # switch now (recovery)

Cross-reference: full systemd unit anatomy, dependencies, socket activation, and timers are in the systemd and journald study guide.

Part 3: Logging and Troubleshooting

Learning objectives - Use journalctl patterns for fast diagnosis - Enable persistent logs - Collect crash information

3.1 journalctl Patterns

journalctl with filters What it does: Queries the systemd journal with filters by unit, time, priority, and boot. Why it matters: The journal holds structured logs from the kernel and every service. Knowing the filter patterns turns 'something's wrong' into a precise diagnosis in seconds. If you skip it: Without filters you scroll through thousands of lines. The right filter (one unit, errors only, this boot) shows you the problem immediately.

$ journalctl -u myapp # one service's logs $ journalctl -u myapp -f # follow live (like tail -f) $ journalctl -b # current boot only $ journalctl -b -1 # previous boot $ journalctl -p err # errors and worse $ journalctl --since '1 hour ago' $ journalctl -u myapp --since today -p warning $ journalctl -k # kernel messages only

3.2 Persistent Logs

Storage=persistent in journald.conf What it does: Makes the journal survive reboots by storing it on disk in /var/log/journal/ instead of volatile /run. Why it matters: By default on many embedded images the journal is volatile — wiped on reboot. If a device crashes and reboots, the logs explaining why are gone. If you skip it: Without persistent logs, every reboot erases the evidence. A device that crash-loops gives you nothing to debug because each crash wipes the prior log.

# /etc/systemd/journald.conf [Journal] Storage=persistent SystemMaxUse=50M # cap disk usage (important on small flash) MaxRetentionSec=1week $ mkdir -p /var/log/journal $ systemctl restart systemd-journald

Flash wear warning Persistent logs write to flash. On a device with heavy logging and a small SD card, this wears the card. Cap SystemMaxUse, rate-limit noisy services, and consider logging to a dedicated data partition. Covered in Part 5.

3.3 Crash Collection

coredumpctl / systemd-coredump What it does: Captures core dumps when a process crashes, and lets you list and extract them for analysis. Why it matters: When the application crashes in the field, a core dump is often the only way to find out why. Without it you have a symptom (it restarted) but no cause. If you skip it: Without crash collection, a field crash is just a gap in the logs. You can't reproduce it on the bench because you don't know what triggered it.

$ coredumpctl list # recent crashes $ coredumpctl info myapp # details of last crash $ coredumpctl dump myapp -o core.dump # extract for gdb # Then analyze (see Deploy/Debug session): $ gdb /usr/bin/myapp core.dump

Cross-reference: gdb analysis of core dumps is in the Deploy and Debug session. journald internals are in the systemd and journald guide.

Part 4: Networking in the Field

Learning objectives - Choose DHCP vs static appropriately - Configure DNS and a basic firewall

4.1 DHCP vs Static

systemd-networkd .network files What it does: Configures interfaces declaratively: DHCP for automatic addressing, or static for fixed addresses. Why it matters: DHCP is right for devices on networks you don't control (customer LANs). Static is right when the device must be reachable at a known address or the network has no DHCP server. If you skip it: Picking wrong causes field problems: a static device on a DHCP network may conflict with another host; a DHCP device that needs a fixed address becomes unreachable when its lease changes.

# DHCP (customer networks) — /etc/systemd/network/10-wan.network [Match] Name=eth0 [Network] DHCP=yes # Static (fixed deployment) — /etc/systemd/network/10-wan.network [Match] Name=eth0 [Network] Address=192.168.10.50/24 Gateway=192.168.10.1 DNS=192.168.10.1

4.2 DNS

systemd-resolved / resolvectl What it does: Manages DNS resolution, optionally with caching, DNS-over-TLS, and DNSSEC. Why it matters: DNS is the most common 'works on my bench, fails in the field' problem. Customer networks have broken, hijacked, or filtered DNS. Caching reduces lookups; DoT prevents tampering. If you skip it: Skipping DNS configuration leaves you at the mercy of whatever DNS the customer's network provides — which may redirect, fail, or leak your queries.

$ resolvectl status # what DNS is actually in use $ resolvectl query example.com # test resolution # Configure DoT in /etc/systemd/resolved.conf: # DNS=1.1.1.1#cloudflare-dns.com # DNSOverTLS=yes

4.3 Firewall

nftables default-deny What it does: Configures the kernel firewall to drop all inbound traffic except what you explicitly allow. Why it matters: A deployed device on an untrusted network should expose nothing it doesn't need. Default-deny is the only safe posture for a security product. If you skip it: A default-allow (or no) firewall means every listening service is reachable by anyone on the network — including services you forgot were running.

# Minimal default-deny (see Network/SSH guide for full ruleset) $ nft add table inet filter $ nft add chain inet filter input { type filter hook input priority 0\; policy drop\; } $ nft add rule inet filter input ct state established,related accept $ nft add rule inet filter input iif lo accept $ nft add rule inet filter input ip saddr 192.168.0.0/16 tcp dport 22 accept

Cross-reference: the Network Essentials and SSH Hygiene guide has the complete firewall ruleset, MTU handling, and field network troubleshooting.

Part 5: Storage Reliability

Learning objectives - Run filesystem checks - Plan for flash wear - Choose a partition strategy

5.1 Filesystem Checks

fsck What it does: Checks and repairs filesystem inconsistencies, typically run automatically at boot on unclean shutdown. Why it matters: Embedded devices lose power without clean shutdown constantly. fsck recovers the filesystem; without it, corruption accumulates until the device won't boot. If you skip it: If fsck isn't configured to run (correct fs-pass in fstab), a corrupted filesystem after a power cut may leave the device unbootable with no automatic recovery.

# In /etc/fstab, the 6th field is fs_passno (fsck order) # /dev/mmcblk0p2 / ext4 defaults 0 1 ← root checked first # /dev/mmcblk0p4 /data ext4 defaults,noatime 0 2 ← data checked second $ fsck /dev/mmcblk0p4 # manual check (unmount first)

5.2 Flash Wear

- noatime mount option — stops updating access times on every read, cutting write traffic dramatically

- Read-only rootfs — the rootfs never writes, so it never wears (see A/B Updates guide)

- Cap journal size — SystemMaxUse in journald.conf limits log writes

- tmpfs for volatile data — /tmp, /var/run on RAM, not flash

- Quality cards — A1/A2-rated cards or eMMC, not bargain SD cards

noatime mount option What it does: Disables writing a timestamp every time a file is read. Why it matters: Default 'relatime' still writes periodically. On a device reading many files, atime updates generate constant background writes that wear flash for no benefit. If you skip it: Without noatime, a read-heavy device wears its flash faster than necessary — shortening the field life of the storage and causing premature failures.

5.3 Partition Strategy

A robust embedded partition layout separates concerns so a problem in one area doesn't take down the whole device:

- Boot (FAT, small) — bootloader, kernel, DTB. Rarely written.

- System A / System B (ext4, read-only) — rootfs, for A/B updates. Never written at runtime.

- Data (ext4, read-write) — config, logs, application state. The only partition that takes runtime writes; the only one that can corrupt on power loss.

Cross-reference: A/B partition design and read-only-rootfs patterns are detailed in the A/B Updates and RO Root guide.

Common pitfalls - enable vs start confusion. start is now; enable is at-boot. Use enable --now to do both. Forgetting enable means the service vanishes after reboot. - Editing shipped units in /lib/systemd/. Wiped on update. Always use systemctl edit to create overrides in /etc. - Volatile journal on a crash-looping device. Each reboot erases the logs. Set Storage=persistent before you need it. - Password SSH on a deployed device. Bots find it within hours. Key-only auth, password auth disabled. - No write/wear strategy on SD-card devices. Heavy logging to a cheap card kills it in months. noatime, capped logs, quality media. - Static IP on a DHCP network (or vice versa). Causes address conflicts or unreachable devices. Match the addressing to the deployment.

Try this yourself - On a test device, systemctl start a service but don't enable it. Reboot. Confirm it's not running. Then enable --now and reboot again. - Use systemctl edit to add Restart=on-failure to a service. Verify the override file location. Kill the service and watch it restart. - Set Storage=persistent in journald.conf. Reboot twice. Run journalctl -b -1 to read the previous boot's logs. - Configure noatime on a data partition in fstab. Remount. Verify with mount ¦ grep noatime. - Set up a default-deny nftables ruleset. From another machine, try to connect to a port you didn't allow. Confirm it's blocked.

References

- systemd and journald study guide (this series) — units, targets, journal internals

- Network Essentials and SSH Hygiene study guide — full SSH hardening and firewall

- A/B Updates and RO Root study guide — partition strategy and read-only rootfs

- systemctl/journalctl man pages: man systemctl, man journalctl

- systemd-networkd: man systemd.network