Performance Checks on Target

Day 4 · Session 4 — measuring boot time, memory, CPU, and I/O

You can't improve what you don't measure. This session covers the four performance dimensions of an embedded device — boot time, memory footprint, CPU usage, and I/O — and the standard tools for each. Every tool is explained with what it measures, why it matters, and what you miss by not measuring it.

How to read this session Four parts, one per dimension: boot time (systemd-analyze), memory (free/ps/smem), CPU (top/mpstat), I/O (iotop/iostat). Each tool gets the what/why/if-skipped treatment plus how to read its output.

Session goals

- Measure and break down boot time with systemd-analyze

- Measure memory footprint accurately (and know why 'free' alone misleads)

- Measure CPU usage and identify what's consuming it

- Find I/O bottlenecks with iotop and iostat

Part 1: Boot Time

Learning objectives - Measure total boot time and its breakdown - Find the services that slow boot

1.1 Why Boot Time Matters

For many embedded products, boot time is a user-facing spec: a door controller that takes 90 seconds to come up after a power blip is a bad product. Boot time is also a proxy for system health — a creeping boot time often signals a service that's started misbehaving.

1.2 systemd-analyze

systemd-analyze What it does: Reports total boot time, split across firmware, bootloader, kernel, and userspace (systemd). Why it matters: It tells you where the time goes. 'Boot is slow' is useless; 'userspace takes 40s and the kernel takes 4s' points you straight at services. If you skip it: Without measuring, you optimize blind — maybe spending days on the kernel when the real cost is one slow service in userspace.

$ systemd-analyze Startup finished in 3.4s (kernel) + 18.6s (userspace) = 22.0s multi-user.target reached after 18.5s in userspace

1.3 systemd-analyze blame

systemd-analyze blame What it does: Lists each service by how long it took to initialize, slowest first. Why it matters: This is the single most useful boot-optimization command. It names the exact services costing you time, in order. If you skip it: Without it, you don't know which of dozens of services is the culprit. The blame list hands you the targets directly.

$ systemd-analyze blame 8.234s network-online.target waiting for DHCP 4.120s myapp.service 2.301s systemd-udev-settle.service 1.005s dev-mmcblk0p2.device ... # The biggest items are your optimization targets. # 'waiting for DHCP' is a classic: the app waits for network # it doesn't actually need at startup.

1.4 systemd-analyze critical-chain

systemd-analyze critical-chain What it does: Shows the dependency chain that actually determined boot time — the critical path, not just the slowest individual units. Why it matters: blame shows slow units, but a slow unit running in parallel may not delay boot. critical-chain shows what's actually on the path that gates reaching the target. If you skip it: Without it you might optimize a slow service that runs in parallel (no boot-time benefit) instead of the one actually blocking the critical path.

$ systemd-analyze critical-chain multi-user.target @18.5s └─myapp.service @14.3s +4.1s └─network-online.target @14.2s └─systemd-networkd-wait-online.service @6.0s +8.2s # Read: myapp waits on network-online, which waits 8.2s on DHCP. # Fixing the network wait shortens the whole chain.

Common boot-time wins for embedded - Don't wait for network if you don't need it at startup. Remove network-online.target from your service's After= if the app can start before the network is up and connect lazily. - Disable services you don't use. systemctl disable anything not needed. Every enabled service costs boot time. - Avoid systemd-udev-settle. It waits for all devices to be probed; rarely needed and often slow. - Mask unnecessary targets. On a headless device, you don't need graphical.target machinery. - Measure after each change. Re-run systemd-analyze to confirm the win is real.

Part 2: Memory Footprint

Learning objectives - Read memory usage correctly - Understand why 'free' alone misleads - Find what's using memory

2.1 free — The Starting Point (and Its Trap)

free -h What it does: Shows total, used, free, and available memory plus buffers/cache. Why it matters: It's the quick 'how much memory is left' check. The key column is 'available' — memory actually reclaimable for new applications. If you skip it: If you read the 'free' column instead of 'available', you'll panic that memory is full when Linux is just using it for cache. Cache is reclaimable; it counts as available.

$ free -h total used free shared buff/cache available Mem: 985Mi 210Mi 120Mi 8Mi 655Mi 720Mi ↑ THIS is what's actually available for new apps. 'free' (120Mi) looks alarming but buff/cache is reclaimable, so 720Mi is really available.

2.2 ps — Per-Process Memory

ps with memory sort What it does: Lists processes; sorted by memory, shows which processes consume the most. Why it matters: free tells you total usage; ps tells you who's responsible. Essential for finding a memory hog or a leak. If you skip it: Without per-process visibility, you know memory is high but not which process to investigate.

# Top memory consumers $ ps aux --sort=-%mem ¦ head -10 # RSS = resident set size (physical RAM actually used) $ ps -eo pid,comm,rss,vsz --sort=-rss ¦ head

2.3 smem — Accurate Shared-Memory Accounting

smem What it does: Reports memory with PSS (Proportional Set Size), which fairly divides shared library memory among the processes that share it. Why it matters: RSS double-counts shared libraries (each process appears to use the full library). PSS gives each process its fair share, so the numbers actually add up to reality. If you skip it: Without PSS, summing per-process RSS hugely overstates total usage because every process counts the shared libc, etc. PSS is the honest number for capacity planning.

$ smem -t -k PID User Command Swap USS PSS RSS 543 root myapp 0 12.0M 14.2M 22.0M ↑ ↑ ↑ private mem ─────┘ │ │ PSS (fair share) ───────┘ │ RSS (overcounts shared) ───────┘ # Install if missing: it's the 'smem' package # USS = truly private (freed if process dies) # PSS = USS + fair share of shared = use this for totals

Part 3: CPU Usage

Learning objectives - See real-time CPU usage - Break down usage per-core and by category

3.1 top — Real-Time Overview

top What it does: Shows real-time CPU and memory usage, per-process, updating continuously. Why it matters: The first tool for 'why is the device slow right now'. Shows the load average and the processes consuming CPU. If you skip it: Without it, 'the device feels sluggish' has no data behind it. top shows immediately whether one process is pegged or the system is just busy.

$ top # Key fields: # load average: 1m, 5m, 15m (>number-of-cores = overloaded) # %Cpu(s): us=user sy=system id=idle wa=io-wait # per process: %CPU, %MEM, TIME+, COMMAND # Batch mode (one snapshot, scriptable): $ top -bn1 ¦ head -15

Reading the CPU summary line matters: high us (user) means application code is busy; high sy (system) means lots of syscalls/kernel work; high wa (io-wait) means the CPU is idle waiting for disk/flash — an I/O bottleneck, not a CPU one (see Part 4).

3.2 mpstat — Per-Core Breakdown

mpstat -P ALL What it does: Reports CPU usage broken down per core and per category (user, system, iowait, idle). Why it matters: On a multi-core device (Pi 3/4 have 4 cores), aggregate CPU% hides imbalance. One core pegged at 100% while three idle shows a single-threaded bottleneck. If you skip it: Without per-core data, a single-threaded process maxing one core looks like '25% CPU' on a 4-core device — you'd never know one core is the bottleneck.

$ mpstat -P ALL 1 CPU %usr %sys %iowait %idle all 24.1 3.2 0.5 72.2 0 98.0 2.0 0.0 0.0 ← core 0 pegged: single-threaded bottleneck 1 0.5 1.0 0.0 98.5 2 0.2 0.5 0.0 99.3 3 0.1 0.3 0.0 99.6 # Install: part of the 'sysstat' package

Part 4: I/O Bottlenecks

Learning objectives - Identify I/O-bound conditions - Find which process is doing the I/O

4.1 Recognizing an I/O Bottleneck

If top shows high wa (io-wait) and low actual CPU usage, the device isn't CPU-bound — it's waiting on storage. This is common on SD-card-based devices where flash is slow. The CPU is idle, twiddling its thumbs while the slow card catches up.

4.2 iostat — Device-Level I/O

iostat -x What it does: Reports per-device I/O statistics: throughput, IOPS, average wait time, and device utilization. Why it matters: Tells you whether storage is the bottleneck and how hard each device is working. The %util column shows how busy the device is; await shows latency. If you skip it: Without it, you see the symptom (high io-wait in top) but not which device or how saturated it is. iostat quantifies it.

$ iostat -x 1 Device r/s w/s rkB/s wkB/s await %util mmcblk0 2.0 45.0 8.0 1800.0 28.5 95.2 ↑ ↑ high latency (ms) ───────────┘ │ near-saturated (%) ─────────────────┘ # %util near 100 = device is the bottleneck # high await = each I/O is slow (typical of cheap SD cards) # Install: 'sysstat' package

4.3 iotop — Per-Process I/O

iotop What it does: Like top, but for disk I/O: shows which processes are reading and writing, in real time. Why it matters: iostat says the device is saturated; iotop says which process is saturating it. Together they pinpoint the cause. If you skip it: Without per-process I/O, you know storage is the bottleneck but not who's responsible — could be your app, could be excessive logging, could be a runaway process.

$ iotop -o # -o = only show processes actually doing I/O PID USER DISK READ DISK WRITE COMMAND 543 root 0.00 B/s 1800 K/s myapp ← heavy writer 102 root 0.00 B/s 120 K/s systemd-journald ← logging # Requires root. Install: 'iotop' package # A heavy writer on an SD-card device is also a flash-wear problem # (see Day-to-Day Operations, storage reliability)

Worked example: 'the device is sluggish but CPU looks low' Symptom: the app is unresponsive, but top shows CPU mostly idle. $ top -bn1 ¦ grep Cpu %Cpu(s): 5.0 us, 2.0 sy, 0.0 ni, 18.0 id, 75.0 wa ↑↑↑↑ 75% io-wait — NOT cpu-bound, io-bound $ iostat -x 1 mmcblk0 ... await 95.0 %util 99.8 ← SD card saturated $ iotop -o 543 root ... 2400 K/s myapp ← myapp writing constantly. Diagnosis: the app is writing to the SD card faster than the card can absorb. The CPU is idle waiting for the slow flash. The fix is in the application (batch writes, write less) or storage (faster media, write to RAM and flush periodically), not in CPU optimization. Lesson: high io-wait means look at I/O, not CPU. Optimizing the code's CPU usage here would achieve nothing.

Common pitfalls - Reading 'free' instead of 'available' in free output. Linux uses free RAM for cache; that cache is reclaimable. 'available' is the real number. - Summing RSS across processes for total memory. Double-counts shared libraries. Use smem's PSS for honest totals. - Trusting aggregate CPU% on a multi-core device. One pegged core reads as 25% on a quad-core. Use mpstat -P ALL. - Optimizing CPU when the problem is I/O. High io-wait means the CPU is idle waiting for storage. Check iostat/iotop before touching code. - Measuring once. Performance varies with load, temperature, and what else is running. Measure under realistic conditions, repeatedly. - Profiling tools not in the production image. top, free are usually present; mpstat, iotop, smem (sysstat/iotop/smem packages) often aren't. Add them to your dev image, not production.

Try this yourself - Run systemd-analyze and systemd-analyze blame on a device. Identify the three slowest services. Pick one and investigate whether it's needed at boot. - Run free -h. Note the difference between 'free' and 'available'. Start a memory-heavy process and watch both change. - Install smem. Compare its PSS total to the sum of RSS from ps. Note how much RSS overcounts. - Run mpstat -P ALL 1 while running a single-threaded CPU-bound program. Watch one core hit 100% while others stay idle. - Run iostat -x 1 and iotop -o while copying a large file. Watch %util climb and identify the process in iotop.

References

- systemd-analyze: man systemd-analyze

- sysstat (mpstat, iostat): man sysstat, sysstat.github.io

- iotop: man iotop

- smem: man smem, selenic.com/smem

- Brendan Gregg's performance tools: brendangregg.com (the definitive Linux performance reference)

- Day-to-Day Operations study guide — storage reliability and flash wear