Updating and Maintaining Devices

Day 4 · Session 5 — update strategies, safety, and long-term maintenance

A device you can't safely update is a liability. This final session covers the trade-offs between package-feed and image-based updates, the safety principles that prevent field bricking, how Yocto's artifacts and versioning support long-term maintenance, and the discipline that keeps a product maintainable over a 10-15 year life.

How to read this session Part 1 contrasts the two update models. Part 2 covers update safety principles (A/B, rollback, power-loss). Part 3 covers how Yocto supports long-term maintenance. Part 4 covers release discipline. A/B updates have a dedicated deep-dive guide; this is the decision-and-principles session.

Session goals

- Choose between package-feed and image-based updates for your product

- Apply the safety principles that prevent field bricking

- Understand how Yocto versioning and artifacts support long-term maintenance

- Adopt release discipline that keeps a product maintainable for years

Part 1: Package Feeds vs Image-Based Updates

Learning objectives - Understand both update models - Choose based on your product's constraints

1.1 The Two Models

There are two fundamentally different ways to update an embedded Linux device:

- Package-feed updates — like apt/dnf on a desktop. The device runs a package manager (opkg, dpkg, rpm) and pulls individual updated packages from a feed server. You update piece by piece.

- Image-based updates — the whole rootfs is replaced as one image (typically via A/B partitions). You ship a complete, tested system image, not individual packages.

1.2 Package-Feed Updates

opkg / package feed What it does: The device runs a package manager that downloads and installs individual updated packages from a network feed. Why it matters: Updates are small (only the changed package) and granular (patch one component without touching others). Good for incremental fixes and large fleets with limited bandwidth. If you skip it: The risk: each device's exact software state diverges over time depending on which updates applied successfully. You lose the 'every device runs an identical, tested image' guarantee. A half-applied update can leave an inconsistent system.

# Enable package management in the image: # IMAGE_FEATURES += "package-management" # PACKAGE_CLASSES = "package_ipk" (or _deb, _rpm) # On the device: $ opkg update # refresh feed list $ opkg list-upgradable # what can be updated $ opkg upgrade myapp # update one package # Feed server is configured in /etc/opkg/*.conf

- Pros: small updates, granular, low bandwidth, fast to deploy a single fix

- Cons: device states diverge, harder to test 'the exact thing in the field', partial-update risk, package manager adds attack surface and storage

1.3 Image-Based Updates

image-based update (A/B) What it does: The entire root filesystem is replaced as a single image, written to an inactive partition, then activated atomically with automatic rollback. Why it matters: Every device runs a known, identical, fully-tested image. There's no divergence, no partial-update state. The atomic switch with rollback makes updates safe against power loss and bad releases. If you skip it: The risk of NOT doing this: with in-place or package updates, you can never be fully certain what's running in the field, and a failed update can brick a device with no automatic recovery.

- Pros: every device identical, fully testable, atomic, safe rollback, no package manager needed in production

- Cons: larger updates (whole rootfs), needs A/B partition space (two copies), more storage

Cross-reference: the A/B Updates and RO Root guide covers image-based updates in depth — SWUpdate, Mender, RAUC, dm-verity, partition design.

1.4 Choosing

Which model for your product? - Security product (like Allegion access control) → image-based. You need certainty about what's running, atomic updates, and guaranteed rollback. The 'every device identical and tested' property is worth the extra storage. - Large fleet, bandwidth-constrained, frequent small fixes → package feeds may win, IF you have strong fleet management to track divergence. - Most modern embedded products → image-based with A/B is the industry direction. Storage is cheap; field reliability and update safety are not. - Hybrid → image-based for the OS, with a separate mechanism for application-only updates (e.g., containerized app updated independently of the base image).

Part 2: Update Safety Principles

Learning objectives - Apply A/B, rollback, and power-loss-tolerance thinking - Design updates that can't brick a device

2.1 A/B (Dual-Bank) Thinking

A/B partition update What it does: Two complete system partitions; the device runs from one while the update is written to the other, then switches over. Why it matters: The running system is never touched during update. If anything goes wrong writing the new image, the device is still running the old one and can retry. If you skip it: Without A/B, you update in place — overwriting the running system. A failure or power loss mid-write leaves no working system to fall back to. Bricked device.

2.2 Rollback Thinking

automatic rollback What it does: After switching to the new image, the device must confirm it booted and works ('commit'); if it doesn't confirm within a timeout, the bootloader reverts to the old image. Why it matters: A new image can pass all bench tests and still fail to boot on a specific field device (different hardware revision, corrupt write, edge-case config). Rollback recovers automatically without a truck roll. If you skip it: Without rollback, a bad update that boots on the bench but fails in the field requires physical access to recover every affected device — potentially thousands of expensive service visits.

# The rollback contract (conceptual): 1. Bootloader sets 'trying new image, attempts=3' 2. New image boots 3. App starts, runs self-checks, calls 'mark boot successful' 4a. If marked successful → new image is now permanent 4b. If NOT marked (crash, hang, failed check) → reboot 5. Bootloader decrements attempts; at 0 → revert to old image # Frameworks (SWUpdate/Mender/RAUC) implement this for you.

2.3 Power-Loss Tolerance

power-fail-safe update design What it does: Every step of the update is designed so that losing power at any moment leaves the device in a recoverable state. Why it matters: Embedded devices lose power at the worst possible moments. The update must be atomic — there's no 'half updated' state, only 'old' or 'new'. If you skip it: An update that isn't power-fail-safe will eventually brick a device when power drops mid-write. At fleet scale, 'eventually' becomes 'regularly'.

- Write then switch — write the entire new image and verify it BEFORE flipping the active-partition flag. The flip is a single atomic operation.

- Verify before activate — checksum/signature-check the written image before trusting it.

- Atomic flag update — the bootloader flag flip must be atomic (single sector write, or redundant copies).

- Journaled or A/B data partition — the writable data partition uses a journaling filesystem so a power cut doesn't corrupt it.

Part 3: Yocto and Long-Term Maintenance

Learning objectives - Use Yocto versioning for reproducible long-term builds - Leverage Yocto artifacts for maintenance

3.1 Reproducible Builds

pinned layers + SRCREV + LTS branch What it does: Every layer is pinned to a specific git commit, every git-fetched recipe to a specific SRCREV, on a Yocto LTS branch. Why it matters: A product shipping for 10-15 years must be rebuildable years from now — to patch a CVE in an old release, for instance. Pinning everything makes the build reproducible from any commit. If you skip it: Without pinning, a rebuild two years later pulls different upstream code and produces a different image. You can't reliably patch an old release because you can't reproduce it.

# Pin the Yocto branch to LTS $ cd poky && git checkout scarthgap # LTS, supported to 2028 # Pin git recipes to specific commits (not AUTOREV) SRCREV = "a1b2c3d4..." # exact commit, reproducible # NOT: SRCREV = "${AUTOREV}" (floats — non-reproducible) # Tag your layer state in git for every release $ git tag release-1.2.3 && git push --tags

3.2 Yocto Artifacts for Maintenance

- License manifest — proves what's in each release for compliance audits years later (see License/SBOM/CVE guide)

- SBOM (SPDX) — the machine-readable inventory, increasingly a legal requirement

- buildhistory — Yocto's record of exactly what changed between builds

- The pinned manifest — the exact layer commits for a release, so you can recreate it

buildhistory What it does: Records the complete contents and metadata of every build (packages, versions, sizes, dependencies, file lists) into a git repository. Why it matters: When investigating 'what changed between release 1.2.2 and 1.2.3' or 'what version was on this returned device', buildhistory has the answer. Essential for long-term support and regression hunting. If you skip it: Without buildhistory, you have no automatic record of how releases differ. Diagnosing a regression introduced three releases ago becomes archaeology.

# Enable in local.conf or distro.conf: INHERIT += "buildhistory" BUILDHISTORY_COMMIT = "1" # After builds, inspect changes: $ buildhistory-diff # Shows package version changes, size changes, dependency changes # between the last two builds

Cross-reference: SBOM, license manifest, and CVE tracking are detailed in the License Compliance, SBOM, and CVE guide. buildhistory provenance is also mentioned in the debug/release discussion.

Part 4: Release Discipline

Learning objectives - Adopt versioning and release practices for long-term products - Understand modern run tooling improvements

4.1 Versioning

- Semantic versioning (MAJOR.MINOR.PATCH) for your product firmware — communicates the nature of each change

- Immutable releases — a version number, once shipped, never changes. A fix is a new version, never a re-release of the same number

- Git-tagged releases — every shipped firmware corresponds to a git tag spanning all layers, so it's fully reproducible

- Build provenance baked in — the running device can report its exact version and git hash (e.g., in /etc/os-release)

version baked into the image What it does: The firmware version and git commit are embedded in the image (e.g. /etc/os-release, or a /etc/build-info file) at build time. Why it matters: When a device reports a problem, the very first question is 'what version are you running'. Baking it in makes that answerable definitively, by the device itself. If you skip it: Without it, you rely on humans correctly recording which version went where — which fails at scale. You end up debugging without knowing what code is actually running.

# Yocto sets these automatically; verify on target: $ cat /etc/os-release NAME="Allegion Linux" VERSION="1.2.3 (scarthgap)" BUILD_ID="20260610-a1b2c3d" ← date + git short hash # Add custom build info via a recipe writing /etc/build-info

4.2 Release Practices

- Every release is reproducible — from a git tag, you can rebuild the identical image

- Every release passes the QA gates — CVE scan, license check, content audit, boot test (see QA Gates guide)

- Every release has an SBOM and license manifest archived — for compliance and audit

- Staged rollout — deploy to a small cohort first, watch for problems, then fleet-wide. A/B + rollback makes this safe

- Accepted-risk register — any known issue shipped anyway is documented with justification and approver

4.3 Modern Run Tooling

The embedded Linux update and operations landscape keeps improving. Worth tracking for long-term products:

- OTA frameworks — SWUpdate, Mender, RAUC continue to mature (covered in the A/B Updates guide)

- Container-based app delivery — some products ship the OS via image-update and the application as a container, updated independently

- Fleet management platforms — Mender, Balena, and others provide dashboards for fleet-wide update status, staged rollouts, and rollback

- Reproducible-build verification — tooling to prove an image matches its source, increasingly relevant for supply-chain security

- Signed everything — signed boot (FIT), signed updates (RAUC mandatory signing), signed SBOMs — the direction of travel for security products

Common pitfalls - In-place updates with no rollback. The classic bricking pattern. A power cut or bad image mid-update leaves no working system. Always A/B with rollback for production. - AUTOREV in production recipes. Makes builds non-reproducible — you can't rebuild an old release to patch it. Pin SRCREV to exact commits. - Not baking version info into the image. 'What version is this device?' becomes unanswerable. Bake it into /etc/os-release at build time. - No buildhistory. Diagnosing what changed between releases becomes guesswork. Enable it from the start; it's nearly free. - Package-feed updates without divergence tracking. Each device drifts to a unique software state. You lose the ability to say 'the field runs exactly this tested configuration'. - Shipping without a rollback-tested update path. Test the rollback, not just the update. An update mechanism whose rollback has never been exercised will fail when you finally need it.

Try this yourself - Enable buildhistory in a build. Make a small change (add a package). Rebuild. Run buildhistory-diff and read what it reports. - Check /etc/os-release on a Yocto device. Identify the version and build ID. Trace how Yocto set them. - Set up SWUpdate or RAUC (from the A/B guide). Build an update. Apply it. Then deliberately make a 'bad' update and watch the rollback trigger. - Pin a recipe's SRCREV to a specific commit. Rebuild. Change the upstream branch and rebuild again — confirm your pinned version is unchanged. - Write down your product's update decision: package-feed or image-based? Justify it against the criteria in Part 1.4.

References

- A/B Updates and RO Root study guide (this series) — SWUpdate, Mender, RAUC, dm-verity in depth

- License Compliance, SBOM, and CVE study guide — compliance artifacts for maintenance

- QA Gates study guide — what makes a release shippable

- Yocto buildhistory: docs.yoctoproject.org/dev-manual/build-quality.html

- opkg: git.yoctoproject.org/opkg

- Reproducible builds: reproducible-builds.org