Yocto and BitBake

A practical study guide for embedded Linux engineers

This document is a practical guide to the Yocto Project ecosystem: Poky, OpenEmbedded, BitBake, layers, recipes, and the build workflow. The focus is on what each concept means, why it exists, where it lives in the source tree, when you use it, and how you use it in practice. Theory is kept brief; commands and worked examples are the priority.

Reading time: roughly 90 minutes if you go straight through. To get real value, you should be at a Linux workstation running through the practical builds (QEMU, BeagleBone, Raspberry Pi 3) as you read. The first build will take 1-3 hours depending on your machine; subsequent incremental builds are minutes.

How to read this document Each major topic answers four questions: What it is, Why it exists, Where it lives, How to use it. Colored callout boxes throughout: pink for “apply at work” guidance, blue for worked examples, green for practice exercises, amber for common pitfalls. The middle of the document walks through three complete builds (QEMU, BeagleBone Black, Raspberry Pi 3) end-to-end.

Prerequisites

- A 64-bit Linux workstation (Ubuntu 22.04 or later recommended) with 50+ GB free disk space, 8+ GB RAM, multi-core CPU

- Comfortable on the Linux command line; can read shell scripts

- Familiarity with git, make, basic C compilation

- Understand at a high level: bootloader, kernel, root filesystem (the four elements of an embedded Linux system)

- You do NOT need prior Yocto experience; this guide assumes none

What you should be able to do after reading

- Explain the difference between Yocto, Poky, OpenEmbedded, and BitBake to a colleague

- Build a working QEMU image from scratch and run it

- Build a working BeagleBone Black image, flash it to SD card, and boot the board

- Build a working Raspberry Pi 3 image and boot it

- Read any recipe (.bb) file and understand what it does

- Add packages to an image via local.conf

- Add a new layer to your build (manually and via bitbake-layers)

- Choose an appropriate Yocto branch/release for a new project

- Inspect any variable in any recipe with bitbake -e

Part 1 — The Yocto Ecosystem

Learning objectives - Distinguish Yocto Project, Poky, OpenEmbedded, BitBake, and OE-Core - Decide when Yocto is the right choice vs alternatives - Understand the input-output model of a Yocto build

1.1 The Four Elements of Embedded Linux

Every embedded Linux system has the same four building blocks. Yocto produces all four.

- Toolchain — cross-compiler and tools to build code for your target architecture. Everything else depends on it.

- Bootloader — initializes the board and loads the kernel (U-Boot is typical).

- Kernel — manages resources, interfaces with hardware.

- Root filesystem — libraries and programs that run once the kernel finishes booting.

Plus a fifth: your application — the code that makes the device do whatever it does (control a lock, drive a motor, transmit data).

1.2 The Names — Yocto, Poky, OpenEmbedded, BitBake

Beginners get confused here. The terms are related but distinct.

- Yocto Project — the umbrella organization. Like Canonical is to Ubuntu, Yocto is to Poky. Founded 2010 by the Linux Foundation, backed by Intel, ARM, NXP, AMD, Facebook, etc.

- Poky — the reference distribution you download. The word “reference” means “example.” Poky is what you actually git clone. It bundles: BitBake + OpenEmbedded-Core + meta-yocto-bsp + meta-poky + documentation.

- OpenEmbedded (OE) — a separate but cooperating project that provides the broader metadata for many architectures and uses cases. Not a distribution — a foundation.

- OpenEmbedded-Core (oe-core) — the common core that Yocto and OpenEmbedded share. This lives in meta/ inside Poky.

- BitBake — the build engine. A task scheduler that reads recipes and executes them. Conceptually similar to make but for entire distributions.

The clearest mental model Yocto Project is the organization. Poky is the example distro you clone. OpenEmbedded-Core is the shared metadata living in meta/. BitBake is the engine that runs the build. You use all of them together; you rarely think about which is which.

1.3 Why Use Yocto

Yocto is the right tool when you need:

- A custom Linux distribution for embedded hardware — not Ubuntu or Debian, but exactly the packages you choose, with exactly the kernel config you need

- Reproducible builds — the same source produces the same binary, today and three years from now

- Long product lifetime — your build infrastructure must survive 10-15 years; Yocto’s versioned releases and layer model support this

- Multiple SKUs from one codebase — different machine configurations, different feature sets, one source tree

- Cross-compilation done right — toolchain, sysroots, and SDK all produced consistently

1.4 When NOT to Use Yocto

Yocto has overhead. Don’t use it when:

- You just need a desktop Linux — use Ubuntu/Fedora directly

- Your device is so small Linux doesn’t fit — you need an RTOS instead

- Your team needs to ship next week with no Yocto experience — the learning curve is real; Buildroot is faster to learn for simpler projects

- You’re prototyping with off-the-shelf images (Raspbian, BalenaOS) and that’s sufficient

1.5 The Input-Output Model

Think of Yocto as a function:

- Input: specifications — hardware (MACHINE), packages (IMAGE_INSTALL), kernel config, layers chosen

- Output: a Linux-based embedded product — kernel image, root filesystem, bootloader, device tree, optionally an SDK

The same specifications give you the same outputs. That reproducibility is the whole point.

Part 2 — Metadata: The Building Blocks

Learning objectives - Identify the four types of metadata and what each is for - Read a recipe file and explain its main directives - Find configuration files, classes, and includes in the source tree

2.1 What Metadata Is

Metadata in Yocto means: the instructions BitBake reads to do a build. Build commands, version pinning, source URLs, patches, configuration. Four file types make up metadata:

- Recipes (.bb, .bbappend) — how to build one software component

- Configuration files (.conf) — variables: machine, distro, user settings

- Classes (.bbclass) — reusable build logic shared across recipes

- Includes (.inc) — shared fragments included by other files

2.2 Recipes (.bb)

A recipe tells BitBake how to build one software package. Filename convention: <name>_<version>.bb. Examples: dhcp_4.4.1.bb, gstreamer1.0_1.16.1.bb.

A typical recipe specifies:

- Where to get the source (HTTP URL, git repo)

- Which patches to apply

- Configuration options (autoconf flags, CMake options)

- Compile options and library dependencies

- How to install (where files go in the rootfs)

- The license

A minimal recipe — anatomy # hello_1.0.bb DESCRIPTION = "Simple hello world C program" SECTION = "examples" LICENSE = "MIT" LIC_FILES_CHKSUM = "file://${COMMON_LICENSE_DIR}/MIT;md5=0835ade..." SRC_URI = "file://hello.c" S = "${WORKDIR}" do_compile() { ${CC} ${LDFLAGS} hello.c -o hello } do_install() { install -d ${D}${bindir} install -m 0755 hello ${D}${bindir} }. Reading this recipe: - SRC_URI — where to fetch source. file:// means it lives alongside the recipe. - S — source directory; ${WORKDIR} is BitBake’s scratch area for this recipe. - do_compile — a task. BitBake automatically runs do_fetch, do_unpack, do_patch, do_configure, do_compile, do_install, do_package. - do_install — installs into ${D} (the package staging dir), which becomes the rootfs layout. To use this in a build, drop it into a layer (more on layers in Part 4) and add IMAGE_INSTALL += "hello" to local.conf.

2.3 .bbappend — Modifying Recipes Without Forking

A .bbappend file extends an existing recipe. You don’t copy and modify the original — you create a small append file that adds to it. The filename must match the original recipe: nginx_1.20.0.bb is extended by nginx_1.20.0.bbappend or nginx_%.bbappend (wildcard for any version).

Typical use: adding a patch to a vendor recipe without forking the whole recipe.

# nginx_%.bbappend in your layer FILESEXTRAPATHS:prepend := "${THISDIR}/files:" SRC_URI += "file://my-custom-config.patch"

2.4 Configuration Files (.conf)

Configuration files set variables — they don’t describe how to build software. Four types:

- Machine (conf/machine/<name>.conf) — hardware configuration. Defines CPU, kernel image type, bootloader, peripherals. Selected via MACHINE= in local.conf.

- Distribution (conf/distro/<name>.conf) — distribution-level policy: package format, init system, feature flags. Selected via DISTRO=.

- Layer (conf/layer.conf) — every layer has one; declares the layer to BitBake.

- User (build/conf/local.conf) — your local choices: which MACHINE, what to install, parallelism, etc.

2.5 Classes (.bbclass)

A class contains reusable build logic. Recipes inherit classes to gain capabilities without duplicating code.

# In a recipe that builds a CMake project: inherit cmake # In a recipe that builds a kernel module: inherit module # In a recipe that builds a systemd service: inherit systemd

Useful built-in classes:

- autotools — handles configure/make/install for autoconf projects

- cmake — handles CMake projects

- meson — handles Meson projects

- kernel — builds Linux kernels

- module — builds out-of-tree kernel modules

- systemd — handles systemd service unit installation

- rm_work — deletes build artifacts after each recipe (saves disk space)

2.6 Includes (.inc)

Shared fragments. If three recipes share the same set of variables, put them in a .inc file and include it from each recipe. Common pattern: split versioned recipes from common metadata.

# gstreamer1.0.inc — common across versions DESCRIPTION = "GStreamer multimedia framework" LICENSE = "LGPL-2.0" ... # gstreamer1.0_1.16.1.bb require gstreamer1.0.inc SRC_URI = "http://...gstreamer-1.16.1.tar.xz" SRC_URI[sha256sum] = "abc123..."

Part 3 — BitBake: The Build Engine

Learning objectives - Run common BitBake commands and read their output - Inspect any variable in any recipe - Trigger and debug individual tasks

3.1 What BitBake Does

BitBake is the engine. It parses metadata, builds a dependency graph, schedules tasks, and runs them in parallel. Conceptually like make but for an entire distribution:

- Reads recipes and follows them: fetches packages, builds them, packages the binaries

- Tracks task dependencies; runs only what changed

- Maximizes CPU usage with parallel execution

- Produces bootable images at the end

3.2 Essential BitBake Commands

# Build an image $ bitbake core-image-minimal # Build a single recipe (just compile, don't make an image) $ bitbake openssh # List all tasks for a recipe $ bitbake -c listtasks openssh # Run a specific task $ bitbake -c clean openssh # remove build artifacts $ bitbake -c cleanall openssh # also remove downloads and sstate $ bitbake -c fetch openssh # just fetch source $ bitbake -c devshell openssh # drop into a shell in the build dir $ bitbake -c menuconfig virtual/kernel # configure kernel interactively # Show all variables for a recipe (huge output — grep for what you want) $ bitbake -e core-image-minimal ¦ grep ^MACHINE= $ bitbake -e core-image-minimal ¦ grep ^IMAGE_INSTALL= $ bitbake -e openssh ¦ grep ^SRC_URI=

3.3 The Task Pipeline

Every recipe goes through a fixed sequence of tasks. The standard ones, in order:

- do_fetch — download source from SRC_URI

- do_unpack — extract archives

- do_patch — apply patches

- do_configure — run configure / cmake / meson

- do_compile — build the software

- do_install — install into staging directory

- do_package — split installed files into packages

- do_package_write_rpm (or _ipk or _deb) — generate the actual package file

For image recipes (core-image-minimal, core-image-sato), additional tasks: do_rootfs (assemble the root filesystem), do_image_* (create disk image formats).

Debugging a failed build Suppose bitbake openssh fails with a compile error. The diagnostic flow: # 1. Find the log $ bitbake -e openssh ¦ grep ^WORKDIR= WORKDIR="/path/to/build/tmp/work/cortexa57-poky-linux/openssh/8.6/" $ ls tmp/work/cortexa57-poky-linux/openssh/8.6/temp/ log.do_compile log.do_configure log.task_order ... # 2. Read the log $ less tmp/work/.../openssh/8.6/temp/log.do_compile # 3. Drop into the build environment to investigate $ bitbake -c devshell openssh # (this gives you a shell with all toolchain env vars set, # in the source directory; you can run 'make' yourself) # 4. After fixing, restart the failed task $ bitbake -c compile openssh. devshell is the single most useful debugging command in BitBake. Internalize this workflow.

Part 4 — Layers: How Yocto Stays Organized

Learning objectives - Explain why layers exist and what problem they solve - List the standard Poky layers and what each provides - Add a layer to your build (manually and via bitbake-layers) - Find new layers from the OpenEmbedded layer index

4.1 What a Layer Is

A layer is a directory of related metadata — recipes, classes, configurations — that can be added to a build. Naming convention: meta-<name>. Putting your entire build in one place would prevent reuse; layers let you split functionality cleanly.

4.2 Why Layers Exist

- Isolation by purpose — BSP, distro, GUI, application: each gets its own layer

- Reusability — a BSP layer for BeagleBone can be reused across many products

- Replaceability — swap one layer for another (different GUI, different distro policy) without changing the rest

- Vendor delivery — chip vendors ship BSP layers; you add them to your build

4.3 The Standard Poky Layers

- meta — OpenEmbedded-Core: the shared foundation. Recipes for ~1000 common software packages. Do not edit this; it’s the upstream baseline.

- meta-poky — Poky-specific distribution metadata: chooses the package manager, init system defaults, distro feature flags. Configures Poky as a distribution.

- meta-yocto-bsp — reference BSP layer. Supports BeagleBone, EdgeRouter, generic 32/64-bit x86. Reference quality only — not for production hardware unless your hardware happens to match.

- meta-selftest — internal Yocto testing infrastructure. Ignore.

- meta-skeleton — template recipes for kernel and BSP development. Useful as a starting point.

4.4 BSP Layers — Vendor-Provided

Real hardware needs a real BSP layer from the silicon vendor:

- meta-ti — Texas Instruments. BeagleBone Black, BeagleBoard, PandaBoard, OMAP, AM335x, AM437x, AM57xx.

- meta-raspberrypi — all Raspberry Pi variants (Pi, Pi 2, Pi 3, Pi 4, Pi Zero).

- meta-intel — Intel x86 boards.

- meta-freescale / meta-imx — NXP i.MX SoCs.

- meta-rockchip — Rockchip SoCs (RK3308, RK3399).

- meta-qcom — Qualcomm.

Most contain conf/machine/<machine>.conf files for each supported board. List supported machines:

$ ls meta-ti/conf/machine/ am335x-evm.conf beaglebone.conf panda.conf ...

4.5 Finding New Layers

Two indices:

- OpenEmbedded layer index (https://layers.openembedded.org) — comprehensive, includes everything

- Yocto Project Compatible Layers (https://www.yoctoproject.org/software-overview/layers/) — fewer layers but tested and validated against Yocto releases

4.6 Adding Layers to Your Build

Two ways. Both modify build/conf/bblayers.conf.

# Method 1: Manual edit of bblayers.conf BBLAYERS ?= " \ /home/user/poky/meta \ /home/user/poky/meta-poky \ /home/user/poky/meta-yocto-bsp \ /home/user/sources/meta-ti \ " # Method 2: bitbake-layers command $ bitbake-layers add-layer /home/user/sources/meta-ti # Show all currently active layers $ bitbake-layers show-layers # Show all recipes (and which layer provides each) $ bitbake-layers show-recipes ¦ less # Show all .bbappend files (recipes being modified) $ bitbake-layers show-appends

Part 5 — Configuration: local.conf and bblayers.conf

Learning objectives - Set the most important variables in local.conf correctly - Understand what belongs in local.conf vs distro vs layer.conf - Configure disk space, parallelism, and image options

5.1 Where Configuration Lives

When you source poky/oe-init-build-env, a build/ directory is created with build/conf/. Two files live there, generated from samples on first use:

- local.conf — your local choices. NOT tracked in source control (typically).

- bblayers.conf — the list of layers active in this build. CAN be tracked.

5.2 Key Variables in local.conf

# === The most important ones === # Target hardware MACHINE = "qemux86-64" # or 'beaglebone-yocto', 'raspberrypi3', etc. # Where to put downloaded source tarballs (reusable across projects) DL_DIR ?= "${TOPDIR}/downloads" # Where to put build artifacts TMPDIR = "${TOPDIR}/tmp" # Parallelism BB_NUMBER_THREADS ?= "${@oe.utils.cpu_count()}" # BitBake task parallelism PARALLEL_MAKE ?= "-j ${@oe.utils.cpu_count()}" # make -j inside each task # Save disk space (deletes per-recipe work dirs after build) INHERIT += "rm_work" # Add extra packages to the image IMAGE_INSTALL:append = " vim openssh htop" # Image format(s) to produce IMAGE_FSTYPES = "ext4 tar.bz2 wic.bz2" # Extra root filesystem space (kilobytes) IMAGE_ROOTFS_EXTRA_SPACE = "512000" # 500 MB extra

5.3 What Belongs Where

- local.conf — things specific to your workstation: paths, parallelism, debug options. Convenient for ad-hoc tweaks. NOT for production policy.

- Layer’s conf/distro/<name>.conf — distribution-level policy: package format, init system, features. Tracked in source control. Production-grade.

- Layer’s conf/machine/<name>.conf — hardware specifics: CPU family, kernel image format, bootloader. Tracked.

- Recipe .bb — how to build one component.

Beyond ad-hoc use: graduate from local.conf to a custom distro layer Adding IMAGE_INSTALL and INHERIT lines to local.conf is fine for experimentation. For a production product, those choices belong in a custom distro layer: - Create meta-mydistro/ with conf/distro/mydistro.conf - Move your IMAGE_INSTALL, distro features, package format choices into that file - In local.conf, only set: MACHINE, DISTRO = "mydistro", and developer-local paths This way the production build is reproducible from git, not from each engineer’s local.conf. Bug-tracked, code-reviewed, auditable.

5.4 Inspecting Variable Values

BitBake variables compose from many sources. The effective value can be surprising. Always check:

# Show all variables and their values for a recipe $ bitbake -e <recipe> ¦ less # Filter for a specific variable $ bitbake -e core-image-minimal ¦ grep ^MACHINE= MACHINE="qemux86-64" $ bitbake -e core-image-minimal ¦ grep ^IMAGE_INSTALL= IMAGE_INSTALL="packagegroup-core-boot ..." $ bitbake -e core-image-minimal ¦ grep ^KERNEL_IMAGETYPE= KERNEL_IMAGETYPE="bzImage" $ bitbake -e core-image-minimal ¦ grep ^IMAGE_FSTYPES=

Part 6 — Images and Packages

Learning objectives - Choose the right image recipe for your project - Add and remove packages from an image - Understand the difference between recipes and packages

6.1 What an Image Is

An image recipe is the top-level recipe that defines a complete root filesystem. It inherits the core-image class and lists packages to include. Image recipes live in meta*/recipes-*/images/*.bb.

$ ls poky/meta*/recipes*/images/*.bb meta/recipes-core/images/core-image-minimal.bb meta/recipes-extended/images/core-image-full-cmdline.bb meta/recipes-graphics/images/core-image-sato.bb meta/recipes-graphics/images/core-image-weston.bb ...

6.2 Common Image Recipes

- core-image-minimal — smallest image that boots. ~10 MB. Good for kernel/bootloader testing.

- core-image-base — minimal + console-only tools.

- core-image-full-cmdline — full command-line tools, no GUI. Most production servers/embedded use a variant of this.

- core-image-sato — X11 window system with Sato theme + GNOME mobile environment. ~200 MB.

- core-image-weston — Wayland-based GUI.

- rpi-hwup-image (from meta-raspberrypi) — RPi hardware-up base.

- rpi-test-image — RPi with additional packages from meta-raspberrypi.

6.3 Packages vs Recipes — The Distinction

Easy to confuse:

- Recipe = build instructions (the .bb file). Singular concept.

- Package = a binary output file (.rpm, .deb, or .ipk). One recipe usually produces MULTIPLE packages.

# Example: the libtiff recipe produces multiple packages $ grep PACKAGES meta/recipes-multimedia/libtiff/tiff_*.bb PACKAGES =+ "tiffxx tiff-utils" # Produces: tiff (the library), tiffxx (C++ bindings), tiff-utils (commands)

To install something in your image, you add the PACKAGE name to IMAGE_INSTALL, not the recipe name:

# In local.conf, to get the lsusb command (from usbutils recipe): IMAGE_INSTALL:append = " usbutils" # To get just the C++ TIFF bindings: IMAGE_INSTALL:append = " tiffxx"

Part 7 — Practical Build: QEMU

Learning objectives - Build a working QEMU image end-to-end - Add packages to the image and rebuild - Run the image and verify it boots

This is the fastest first build because QEMU needs no hardware. Plan for 1-3 hours the first time (mostly downloading sources and building the toolchain); subsequent builds are minutes.

7.1 First-Time Setup

# Install host prerequisites (Ubuntu 22.04) $ sudo apt-get install gawk wget git diffstat unzip texinfo \ gcc-multilib build-essential chrpath socat cpio python3 \ python3-pip python3-pexpect xz-utils debianutils \ iputils-ping python3-git python3-jinja2 libegl1-mesa \ libsdl1.2-dev pylint3 xterm # Clone Poky $ mkdir -p ~/yocto && cd ~/yocto $ git clone git://git.yoctoproject.org/poky $ cd poky # Choose a release (Scarthgap is the current LTS as of 2026) $ git checkout scarthgap

7.2 The Build

# Initialize the build environment $ cd ~/yocto $ source poky/oe-init-build-env build_qemu # (you are now in build_qemu/; conf/local.conf has been created) # Verify MACHINE is qemux86-64 (default) $ grep ^MACHINE conf/local.conf MACHINE ?= "qemux86-64" # Add useful packages to the image (optional) $ echo 'IMAGE_INSTALL:append = " vim usbutils htop"' >> conf/local.conf # Build (this takes 1-3 hours first time) $ time bitbake core-image-minimal

7.3 Where the Output Lives

$ ls tmp/deploy/images/qemux86-64/ bzImage-qemux86-64.bin core-image-minimal-qemux86-64.ext4 core-image-minimal-qemux86-64.tar.bz2 core-image-minimal-qemux86-64.wic.qcow2 modules-qemux86-64.tgz ...

7.4 Running in QEMU

# Launch QEMU with the built image $ runqemu core-image-minimal # Without graphical window (useful in SSH sessions) $ runqemu qemux86-64 nographic # Login: 'root' (no password by default) # Exit QEMU: Ctrl-A then X (in nographic mode)

7.5 Building for ARM (qemuarm)

Same source tree, different MACHINE.

# Edit local.conf $ sed -i 's/^MACHINE.*/MACHINE = "qemuarm"/' conf/local.conf $ bitbake core-image-minimal $ runqemu qemuarm

The same source produces an ARM image. The toolchain is rebuilt for ARM automatically. This is the leverage of Yocto — one source tree, multiple architectures.

Part 8 — Practical Build: BeagleBone Black

Learning objectives - Build an image for real ARM hardware - Prepare an SD card with partitions and filesystems - Boot the board and connect via serial

8.1 The Board

- SoC — Texas Instruments AM335x (ARM Cortex-A8)

- RAM — 512 MB DDR3

- Storage — 4 GB eMMC + microSD slot

- I/O — USB host + client, Ethernet, HDMI, 2x 46-pin headers (I2C, SPI, UART, GPIO)

BeagleBone Black is a reference board of Yocto Project — the meta-yocto-bsp layer supports it directly.

8.2 The Build

$ cd ~/yocto $ source poky/oe-init-build-env build_bbb # Set MACHINE to BeagleBone $ sed -i 's/^MACHINE.*/MACHINE ?= "beaglebone-yocto"/' conf/local.conf # (optional) save disk space $ echo 'INHERIT += "rm_work"' >> conf/local.conf # Build a minimal image $ bitbake core-image-minimal # Output: $ ls tmp/deploy/images/beaglebone-yocto/ MLO # first-stage bootloader u-boot.img # second-stage bootloader zImage # kernel am335x-boneblack.dtb # device tree core-image-minimal-beaglebone-yocto.tar.bz2 # rootfs core-image-minimal-beaglebone-yocto.wic # full disk image

8.3 BeagleBone Boot Stages

The AM335x boots in four stages because internal RAM is only 128 KB — too small for a full U-Boot. Each stage unlocks more of the device:

Stage 1: ROM Code (hardcoded by TI, not modifiable) ├─ configures stack, PLL, system clocks ├─ watchdog timer setup (3-min) └─ checks boot sources (eMMC, SD, UART, USB) and loads SPL ↓ Stage 2: SPL / MLO (Secondary Program Loader — cut-down U-Boot) ├─ must fit in 128 KB internal RAM ├─ initializes external DDR3 RAM └─ loads full U-Boot into DDR ↓ Stage 3: U-Boot (u-boot.img) ├─ serial console; environment variables in uEnv.txt ├─ loads kernel (zImage) and device tree (.dtb) └─ starts the kernel ↓ Stage 4: Linux Kernel └─ initializes, mounts rootfs, runs init (systemd)

8.4 Preparing an SD Card

The traditional way (manual partitioning):

# 1. Identify the device (CAREFULLY — wrong device = data loss) $ lsblk # Likely /dev/sdb for an external SD card reader # 2. Unmount any auto-mounted partitions $ sudo umount /dev/sdb1 2>/dev/null $ sudo umount /dev/sdb2 2>/dev/null # 3. Use fdisk to partition $ sudo fdisk /dev/sdb d (delete existing partitions) n p 1 (new primary partition 1) +32M (32 MB for boot) n p 2 (new primary partition 2) <enter> (use remaining space for rootfs) a 1 (mark partition 1 as bootable) t 1 c (set partition 1 type to W95 FAT32 LBA — essential) w (write changes) # 4. Format $ sudo mkfs.vfat -n "BOOT" /dev/sdb1 $ sudo mkfs.ext4 -L "ROOT" /dev/sdb2

8.5 Copying Images to the Card

# Mount the partitions (often auto-mounted to /media/$USER/...) $ sudo mount /dev/sdb1 /mnt/boot $ sudo mount /dev/sdb2 /mnt/root # Copy bootloader and kernel to BOOT $ cd tmp/deploy/images/beaglebone-yocto $ sudo cp MLO /mnt/boot/ $ sudo cp u-boot.img /mnt/boot/ $ sudo cp zImage /mnt/boot/ $ sudo cp am335x-boneblack.dtb /mnt/boot/ # Extract rootfs $ sudo tar -xf core-image-minimal-beaglebone-yocto.tar.bz2 -C /mnt/root/ # Sync and unmount $ sync $ sudo umount /mnt/boot /mnt/root

The shortcut: dd a .wic image If your build produces a .wic file (full disk image), you can skip all of the partitioning above: $ cd tmp/deploy/images/beaglebone-yocto $ sudo dd if=core-image-minimal-beaglebone-yocto.wic \ of=/dev/sdb \ bs=4M status=progress $ sync. The .wic already contains the partition table, FAT32 boot, ext4 root, bootloader, kernel, and rootfs. One command, fully bootable. Verify your build produces a .wic by checking IMAGE_FSTYPES in local.conf includes wic.

8.6 Serial Console Setup

BeagleBone’s primary console is serial UART, not HDMI. You need a USB-TTL adapter (PL2303) connected to the J1 header:

J1 Pin USB-TTL 1 GND 4 RX (board) → TX (adapter) 5 TX (board) → RX (adapter) # Configure minicom (one-time) $ sudo minicom -s Serial port setup: Serial Device: /dev/ttyUSB0 Bps/Par/Bits: 115200 8N1 Hardware Flow Control: No Software Flow Control: No Save setup as dfl Exit # Connect $ sudo minicom

8.7 Booting and First Login

Insert the SD card, hold the boot button (S2) near the SD slot, and apply power. After a few seconds:

U-Boot SPL 2021.10 ... Trying to boot from MMC1 U-Boot 2021.10 ... ... Starting kernel ... [ 0.000000] Booting Linux on physical CPU 0x0 [ 0.000000] Linux version 5.15.x ... ... Poky (Yocto Project Reference Distro) 4.0 beaglebone-yocto /dev/ttyS0 beaglebone-yocto login: root root@beaglebone-yocto:~#

Common pitfalls - Partition 1 type must be FAT32 (LBA), type 0x0c. Without this, ROM code won’t recognize the partition as bootable. The most common reason BeagleBone won’t boot from a freshly prepared SD card. - eMMC takes priority over SD by default. If the eMMC contains an old image, you need to either hold the boot button during power-on, or erase the eMMC (mmc dev 1; mmc erase 0 512 from U-Boot prompt). - Forgetting sync after dd. Without sync, data may still be buffered when you remove the card; the boot fails mysteriously. - Wrong device in dd. Triple-check /dev/sdb (or wherever your card is). dd if=image.wic of=/dev/sda will destroy your main hard disk. - Serial cable miswired. RX/TX must cross. Adapter TX to board RX, adapter RX to board TX. Easy to get wrong; if you see no output, swap them.

Part 9 — Practical Build: Raspberry Pi 3

Learning objectives - Add a third-party BSP layer (meta-raspberrypi) - Build for non-reference hardware - Understand RPi-specific boot process and configuration

9.1 The Board

- SoC — Broadcom BCM2837 (4x ARM Cortex-A53 @ 1.2 GHz)

- GPU — Broadcom VideoCore IV (handles boot!)

- RAM — 1 GB LPDDR2

- Networking — 10/100 Ethernet, 2.4 GHz 802.11n WiFi, Bluetooth 4.1

- Storage — microSD only

- I/O — HDMI, USB 2.0 x4, 40-pin GPIO header

9.2 Why meta-raspberrypi

Poky’s meta-yocto-bsp does NOT support Broadcom BCM SoCs. You need a third-party BSP layer.

# Clone meta-raspberrypi alongside Poky $ cd ~/yocto $ git clone git://git.yoctoproject.org/meta-raspberrypi $ cd meta-raspberrypi && git checkout scarthgap && cd .. # meta-raspberrypi depends on meta-openembedded (extra recipes) $ git clone git://git.openembedded.org/meta-openembedded $ cd meta-openembedded && git checkout scarthgap && cd ..

9.3 Setting Up the Build

$ cd ~/yocto $ source poky/oe-init-build-env build_pi3 # Add the layers (depended-on first, then meta-raspberrypi) $ bitbake-layers add-layer ../meta-openembedded/meta-oe $ bitbake-layers add-layer ../meta-openembedded/meta-multimedia $ bitbake-layers add-layer ../meta-openembedded/meta-networking $ bitbake-layers add-layer ../meta-openembedded/meta-python $ bitbake-layers add-layer ../meta-raspberrypi # Verify $ bitbake-layers show-layers # Set MACHINE $ echo 'MACHINE = "raspberrypi3"' >> conf/local.conf # (optional) enable UART for serial console $ echo 'ENABLE_UART = "1"' >> conf/local.conf # List available images from meta-raspberrypi $ ls ../meta-raspberrypi/recipes-*/images/ rpi-hwup-image.bb rpi-test-image.bb # Build $ bitbake rpi-hwup-image

9.4 Raspberry Pi 3 Boot Process

Completely different from BeagleBone. The GPU boots FIRST, not the CPU.

Stage 1: On-chip ROM (GPU) ├─ CPU is held in reset ├─ GPU runs first — only it has access at this point └─ GPU ROM reads bootcode.bin from SD card into L2 cache ↓ Stage 2: bootcode.bin (still on GPU) ├─ enables external SDRAM └─ loads start.elf into SDRAM ↓ Stage 3: start.elf (the GPU's operating system VCOS) ├─ reads config.txt for system configuration ├─ reads cmdline.txt for kernel command line ├─ loads kernel7.img and the device tree blob (.dtb) └─ finally enables the ARM CPU and starts the kernel ↓ Stage 4: Linux Kernel (ARM CPU) └─ normal Linux boot

Key files on the SD card’s FAT partition:

- bootcode.bin — GPU first-stage bootloader (closed-source, from Broadcom)

- start.elf — GPU firmware (closed-source)

- config.txt — hardware configuration: HDMI, memory split, UART

- cmdline.txt — Linux kernel command line

- kernel7.img — the Linux kernel image

- *.dtb — device tree blob

9.5 Flashing the SD Card

RPi3 builds produce a .rpi-sdimg file containing partition table, FAT boot, ext4 root — everything ready to dd:

$ cd tmp/deploy/images/raspberrypi3 $ ls *.rpi-sdimg rpi-hwup-image-raspberrypi3.rpi-sdimg $ lsblk # identify your SD card $ sudo dd if=rpi-hwup-image-raspberrypi3.rpi-sdimg \ of=/dev/sdb \ bs=4M status=progress $ sync

9.6 Serial Console

USB-TTL adapter on the GPIO header. Wiring for J8 connector:

J8 Pin USB-TTL Function 6 GND 8 RX (board) → TX (adapter) 10 TX (board) → RX (adapter)

Same minicom setup as BeagleBone, but use /dev/ttyUSB0 at 115200 8N1.

9.7 Variants

Same build process, different MACHINE variable for other Pi models:

raspberrypi # original Pi raspberrypi2 # Pi 2 raspberrypi3 # Pi 3 (32-bit) raspberrypi3-64 # Pi 3 (64-bit kernel) raspberrypi4 # Pi 4 (32-bit) raspberrypi4-64 # Pi 4 (64-bit) raspberrypi0 # Pi Zero raspberrypi0-wifi # Pi Zero W

Part 10 — Releases and Branch Selection

Learning objectives - Choose an appropriate Yocto branch for a new project - Understand LTS vs stable lifecycle differences - Match BSP layer branches to your Poky branch

10.1 Naming

Yocto releases are numbered Major.Minor.patch (e.g., 4.0.13). Poky uses the same numbering plus code names (e.g., scarthgap, kirkstone, dunfell).

- Major changes — may break compatibility with previous releases

- Minor changes — significant features, no compatibility break

- Patch changes — bug fixes, security updates

Branches in git use the code names. Code names are alphabetical: dunfell, gatesgarth, hardknott, honister, kirkstone, langdale, mickledore, nanbield, scarthgap, styhead.

10.2 Release Schedule

- Major releases — every six months, in April and October

- Minor releases (point releases) — no fixed schedule; driven by accumulated fixes

10.3 Lifecycle: Stable vs LTS

- Stable releases — maintained 7 months. Then community-supported or EOL.

- LTS releases — maintained initially 2 years (extended to 4+ for active LTS). Pre-announced as LTS. Use LTS for production.

As of 2026, active LTS releases include Scarthgap (5.0, April 2024, LTS through April 2028) and Kirkstone (4.0, April 2022, extended LTS).

10.4 How to Choose a Branch

For a new project:

- Start with the latest LTS. Not the absolute newest stable — the most recent LTS.

- Verify your BSP layer supports it. A meta-rockchip that’s only on kirkstone branch can’t be used with a scarthgap Poky checkout.

- Verify your build host supports it. A 2018 Ubuntu may lack tools needed for the newest Yocto.

- Check meta-openembedded and any third-party layers. Same branch name across all layers.

The compromise problem Sometimes you need a layer that hasn’t been updated past kirkstone, but you want scarthgap features. Real-world options: (1) port the layer forward yourself, (2) stay on kirkstone, (3) drop the layer. Discovering this incompatibility early in a project is much better than discovering it 6 months in.

Part 11 — Audit Patterns and Troubleshooting

Learning objectives - Inspect what BitBake is building and where - Find which recipes provide which packages and binaries - Manage disk space across long-running projects

11.1 Inspection Commands

# Show all layers currently active $ bitbake-layers show-layers # Show every recipe and which layer provides it $ bitbake-layers show-recipes ¦ less # Find recipes with a name pattern $ bitbake-layers show-recipes 'openssl*' # Find all .bbappend files (recipes being modified) $ bitbake-layers show-appends # What package provides a given binary in the image? $ oe-pkgdata-util find-path /usr/bin/lsusb usbutils: /usr/bin/lsusb # What files does a package install? $ oe-pkgdata-util list-pkg-files usbutils # What packages does an image include? $ cat tmp/deploy/images/<machine>/<image>.manifest

11.2 Disk Space Management

Yocto builds are large. A single full build can consume 30-100 GB:

- downloads/ — source tarballs (10-20 GB typical). Reusable across builds; share via DL_DIR.

- sstate-cache/ — shared state cache (10-30 GB). Reusable; speeds incremental rebuilds dramatically. Share via SSTATE_DIR.

- tmp/ — per-build artifacts (20-50 GB). Disposable; deleted by bitbake -c cleanall or manually.

# Save disk space during builds $ echo 'INHERIT += "rm_work"' >> conf/local.conf # rm_work deletes per-recipe work dirs after each recipe completes # Tradeoff: harder to debug failed recipes # Exclude a recipe from rm_work (keep its work dir for debugging) $ echo 'RM_WORK_EXCLUDE += "core-image-minimal"' >> conf/local.conf # Share downloads and sstate across multiple builds $ echo 'DL_DIR = "/home/shared/yocto-downloads"' >> conf/local.conf $ echo 'SSTATE_DIR = "/home/shared/yocto-sstate"' >> conf/local.conf # Clean up after a build $ rm -rf tmp/ # removes all per-build artifacts # Downloads and sstate survive; rebuild is faster than from scratch

11.3 Common Build Failures

Common pitfalls - “Nothing PROVIDES X” — a recipe or package isn’t available. Either the layer isn’t added (bitbake-layers add-layer), or the recipe name is wrong (bitbake-layers show-recipes to find it). - “Required build target failed to build” — a recipe failed. Read the log: bitbake -e <recipe> ¦ grep ^WORKDIR= to find the work dir, then less tmp/work/.../temp/log.do_<task>. - Disk full mid-build — typically tmp/ overflows. Set INHERIT += "rm_work" going forward; clean tmp/ now. - “CheckSum mismatch” — a downloaded tarball doesn’t match the recipe’s expected checksum. Either the upstream changed (rare), the download is corrupt (retry), or the recipe is wrong (rare). - “Task hash mismatch” — sstate cache is inconsistent with metadata changes. Run bitbake -c cleansstate <recipe> to force a rebuild. - Slow first build — expected. The first build downloads the entire upstream source and builds the toolchain from scratch. 1-3 hours on a modern workstation. Subsequent incremental builds are minutes. - BSP layer branch mismatch — you cloned meta-raspberrypi on master but Poky on scarthgap. They’re incompatible. git checkout scarthgap on every layer.

11.4 The Yocto Build Health Audit

For an active project, periodically check:

- Layer branches match — every layer in bblayers.conf on the same branch as Poky

- Disk usage — du -sh downloads sstate-cache tmp

- Build reproducibility — bitbake -e <image> output diff between two checkouts of the same commit

- License compliance — tmp/deploy/licenses/ contains per-image license manifests

- Image content — <image>.manifest lists every package installed; review for unexpected additions

- Build time trends — if builds keep getting slower, something is being recomputed when it shouldn’t be (sstate problem)

Closing and References

Glossary

- BBAPPEND — file extending an existing recipe; lets you patch upstream recipes without forking

- BitBake — the build engine; reads metadata and executes tasks

- BSP — Board Support Package; metadata describing hardware (kernel config, bootloader, machine .conf)

- bblayers.conf — lists the layers active in this build

- Class (.bbclass) — reusable build logic that recipes inherit

- Configuration file (.conf) — sets variables; doesn’t describe building

- Image — a top-level recipe defining a complete root filesystem

- Layer — directory of related metadata; meta-<name> convention

- local.conf — per-developer build settings (MACHINE, IMAGE_INSTALL, paths)

- MACHINE — the target hardware; selects a conf/machine/<name>.conf from some BSP layer

- Metadata — the build instructions BitBake reads (recipes, classes, configs, includes)

- OpenEmbedded-Core (oe-core) — the shared metadata foundation, in meta/

- Package — a binary output file (.rpm, .deb, .ipk); one recipe produces many

- Poky — the reference distribution; what you git clone

- Recipe (.bb) — instructions for building one software component

- rm_work — class that deletes per-recipe work dirs after build (saves space)

- sstate — shared state cache; speeds incremental rebuilds

- Task — a single unit of work (fetch, compile, install, etc.) within a recipe

- Yocto Project — the organization (Linux Foundation working group)

- .wic — full disk image format; partition table + filesystems + bootloader in one file

References

- Yocto Project documentation — docs.yoctoproject.org. The definitive reference.

- Yocto Project Reference Manual — every variable, every class, every directive

- Yocto Project Mega-Manual — all manuals combined; searchable single page

- BitBake User Manual — docs.yoctoproject.org/bitbake/

- OpenEmbedded layer index — layers.openembedded.org

- Yocto Project Compatible Layers — yoctoproject.org/software-overview/layers/

- Mailing lists — yocto@lists.yoctoproject.org for general; bitbake-devel for build engine

- Books — “Embedded Linux Systems with the Yocto Project” (Streif), “Embedded Linux Projects Using Yocto Project Cookbook” (González)

What to Do This Week

One concrete action Pick one of the three practical builds (QEMU, BeagleBone Black, or Raspberry Pi 3) and complete it end-to-end. Don’t just read — actually do it. - Set up the prerequisites on your workstation - Clone Poky and check out the latest LTS branch - Source the environment, configure local.conf, run bitbake core-image-minimal - (For BBB/RPi3) Flash an SD card and boot the board - Add a package via IMAGE_INSTALL and rebuild - Use bitbake -e to inspect three variables: MACHINE, IMAGE_INSTALL, KERNEL_IMAGETYPE - Use bitbake-layers show-layers and bitbake-layers show-recipes to explore Plan a full day for the first build (most of it waiting for the build to finish). Subsequent builds are minutes. The hands-on experience makes all the theory in this document concrete.

Combined with the previous study guides on capabilities, cgroups, systemd, journald, and network/SSH hygiene, you now have the foundation for production embedded Linux: how to build the image (Yocto), how to harden the services it runs (capabilities, cgroups), how to manage their lifecycle (systemd), how to observe them (journald), and how to expose them to networks safely (network and SSH hygiene). Six pillars; together they cover most of what serious embedded Linux engineering requires.