Device Tree and Debugging

Writing, reading, and troubleshooting device trees in embedded Linux

The Device Tree is how Linux learns about hardware it can't discover at runtime. On x86 the BIOS does this; on ARM embedded boards, the Device Tree Source (DTS) file does it. This document covers DTS syntax, the compilation and boot flow, overlays, common debugging techniques, and Yocto integration.

How to read this document Parts 1-3 are concept: what a device tree is, the syntax, and how to compile and boot it. Parts 4-6 are practical: overlays, debugging, and Yocto integration. Pink callouts are apply-at-work guidance, blue are worked examples, amber are pitfalls.

Prerequisites

- Basic Linux device driver concepts (how the kernel finds hardware)

- Comfortable with C-like syntax

- Some experience with a real ARM board (BeagleBone Black or Raspberry Pi)

What you should be able to do after reading

- Read any .dts/.dtsi file and understand what hardware it describes

- Write a simple DTS node for a custom peripheral

- Compile, apply, and boot a device tree

- Write and apply a Device Tree Overlay

- Debug device tree problems using kernel logs and /proc/device-tree

- Integrate custom DTS files into a Yocto build

Part 1: What a Device Tree Is

Learning objectives - Explain why device trees exist - Identify the files involved in the DT workflow

1.1 The Problem Device Trees Solve

On a standard PC, the BIOS/UEFI enumerates hardware and tells the OS what's present. On ARM embedded boards, there's no BIOS. The kernel is loaded directly and must somehow know: which UART is the console, which I2C bus has an accelerometer, which GPIO drives the LED, where is the Ethernet controller in memory.

Before Device Trees (pre-2011 ARM Linux), this was hardcoded. Every board had C code in arch/arm/mach-<board>/ that hardcoded all the addresses, IRQ numbers, and peripheral configurations. With thousands of embedded boards, this became unmaintainable.

Device Tree solves this by describing hardware in a separate data structure that the bootloader passes to the kernel at boot. The kernel reads it; drivers bind to nodes; no hardcoding needed.

1.2 The Three File Types

- DTS (Device Tree Source) : the human-readable source file. Extension .dts. One file per board.

- DTSI (Device Tree Source Include) : shared fragments included by multiple DTS files. Extension .dtsi. Typically describes the SoC (CPU, peripherals) while the .dts describes the board.

- DTB (Device Tree Blob) : the compiled binary. Extension .dtb. What the bootloader actually passes to the kernel.

Part 2 : DTS Syntax

Learning objectives - Read and write DTS nodes - Understand properties, phandles, and labels

2.1 The Basic Structure

Annotated minimal DTS file /dts-v1/; // ^^^ mandatory header / { // The root node: represents the system compatible = "raspberrypi,3-model-b", "brcm,bcm2837"; // ^^^ what board/SoC this is; drivers match on this string model = "Raspberry Pi 3 Model B"; // ^^^ human-readable board name (shows in dmesg) #address-cells = <1>; // addresses in child nodes are 1 cell (32-bit) #size-cells = <1>; // sizes in child nodes are 1 cell (32-bit) memory@0 { device_type = "memory"; reg = <0x0 0x40000000>; // base=0x0, size=1GB }; cpus { #address-cells = <1>; #size-cells = <0>; cpu@0 { device_type = "cpu"; compatible = "arm,cortex-a53"; reg = <0>; // CPU index }; }; uart0: serial@7e201000 { // ^^^ label: name (label lets other nodes reference this) compatible = "brcm,bcm2835-aux-uart"; reg = <0x7e201000 0x40>; // base address, size in bytes interrupts = <1 29>; status = "okay"; // ^^^ okay=enabled, disabled=not active }; };.

2.2 Key Properties

- compatible — most important property. A list of strings (most-specific first) the driver matches against. Format: "vendor,model".

- reg — register address and size. Format: <base size>. Number of cells determined by parent's #address-cells and #size-cells.

- interrupts — interrupt specification. Format depends on the interrupt controller (the interrupts-extended property handles complex cases).

- status — "okay" to enable, "disabled" to disable. A node with status = "disabled" is described but the driver doesn't bind.

- clocks — clock sources the device needs.

- pinctrl-* — pin mux configuration.

- Labelsuart0: before a node name creates a label. Other nodes reference it with &uart0. Used heavily in overlays.

2.3 DTSI: Shared Fragments

A typical embedded board's DT is split across files:

# SoC file (describes all peripherals at their hardware addresses) bcm2837.dtsi # ARM cores, GIC, memory map, all peripherals # Board file (selects which peripherals are active, adds board-specific hardware) bcm2837-rpi-3-b.dts #include "bcm2837.dtsi" // Then enables/configures the specific peripherals used on this board

2.4 Modifying Existing Nodes with &label

You don't need to rewrite a whole node to change it. Reference it by label and add/override properties:

// Enabling UART and configuring it for 115200 baud &uart0 { status = "okay"; current-speed = <115200>; }; // Adding an I2C device on an existing bus &i2c1 { status = "okay"; clock-frequency = <400000>; // 400 KHz fast mode bmp280@76 { compatible = "bosch,bmp280"; reg = <0x76>; // I2C address }; };

Part 3: Compiling and Booting

Learning objectives - Compile DTS to DTB - Verify a DTB loaded by U-Boot - Inspect the live device tree at runtime

3.1 The Compiler: dtc

# Compile DTS to DTB $ dtc -I dts -O dtb -o myboard.dtb myboard.dts # Decompile DTB back to DTS (for inspection) $ dtc -I dtb -O dts -o decoded.dts myboard.dtb # Include preprocessor (for #include in .dts files) $ cpp -I /path/to/kernel/include -x assembler-with-cpp myboard.dts \ ¦ dtc -I dts -O dtb -o myboard.dtb # Install on Ubuntu/Debian $ sudo apt-get install device-tree-compiler

3.2 Checking DTB in U-Boot

# At U-Boot prompt, after loading DTB to memory: => fdt addr 0x88000000 # tell U-Boot where the DTB is => fdt print # print entire tree => fdt print / # print root node => fdt print /uart0 # print a specific node => fdt get value status /uart0 status # get one property

3.3 Inspecting the Live Device Tree

Once Linux boots, the device tree is exposed under /proc/device-tree/ (symlinked to /sys/firmware/devicetree/base/). This is the actual DTB the kernel parsed.

# List all nodes $ ls /proc/device-tree/ # Read a property (binary; use xxd for hex or strings for text) $ cat /proc/device-tree/compatible ¦ tr '\0' '\n' raspberrypi,3-model-b brcm,bcm2837 # Find a specific node $ find /proc/device-tree -name compatible -exec sh -c \ 'echo "$1: "; cat "$1"' _ {} \; 2>/dev/null ¦ less # Check if a device was claimed by a driver $ ls /sys/bus/platform/devices/ $ ls /sys/bus/i2c/devices/

Part 4: Device Tree Overlays

Learning objectives - Write and apply an overlay - Use RPi overlays to enable peripherals - Understand overlay scope and limitations

4.1 What Overlays Are

A Device Tree Overlay is a partial DTB that modifies the base DTB at boot time (by the bootloader) or at runtime (via configfs). Instead of modifying the main DTS and recompiling, you write a small overlay that adds or modifies nodes.

Raspberry Pi made overlays mainstream , all the dtoverlay= entries in config.txt are DTB overlay files from /boot/overlays/.

4.2 Writing an Overlay

Worked example: overlay to enable I2C and add a sensor /dts-v1/; /plugin/; // ^^^ both these lines are mandatory for overlays / { compatible = "brcm,bcm2835"; // ^^^ which base DTB this overlay is for fragment@0 { target = <&i2c1>; // target the i2c1 node by phandle __overlay__ { status = "okay"; clock-frequency = <400000>; bmp280@76 { compatible = "bosch,bmp280"; reg = <0x76>; }; }; }; }; # Compile it: $ dtc -I dts -O dtb -o bmp280.dtbo bmp280-overlay.dts # Apply on Raspberry Pi via config.txt: dtoverlay=bmp280. The compiled .dtbo file goes in /boot/overlays/. The bootloader merges it with the base DTB before handing off to the kernel.

4.3 Runtime Overlays via configfs

# Mount configfs (if not already) $ mount -t configfs none /sys/kernel/config # Apply an overlay at runtime $ mkdir /sys/kernel/config/device-tree/overlays/my-overlay $ cat my-overlay.dtbo > /sys/kernel/config/device-tree/overlays/my-overlay/dtbo # Remove an overlay $ rmdir /sys/kernel/config/device-tree/overlays/my-overlay # Requires CONFIG_OF_OVERLAY=y and CONFIG_CONFIGFS_FS=y

Part 5: Debugging Device Trees

Learning objectives - Use dmesg and sysfs to diagnose DT problems - Fix the most common DT errors

5.1 The Debug Toolkit

# 1. Check if a device was found and probed $ dmesg ¦ grep -i 'of_platform' $ dmesg ¦ grep 'bmp280' # look for your device's driver name # 2. Check if the driver bound to the device $ ls /sys/bus/i2c/devices/ # should show 1-0076 for bmp280 at addr 0x76 $ cat /sys/bus/i2c/devices/1-0076/name # 3. Check if the node exists in the live tree $ find /proc/device-tree -name 'bmp280*' # 4. Check the node's status property $ cat /proc/device-tree/i2c@7e804000/bmp280@76/status # 5. Check compatible string vs driver $ cat /proc/device-tree/i2c@7e804000/bmp280@76/compatible bosch,bmp280 # Then check the kernel's registered drivers: $ grep 'bmp280' /sys/bus/i2c/drivers/bmp280/ # 6. Verbose boot with DT debug messages # Add to kernel command line: # of_devmgmt=debug

5.2 Common Errors and Fixes

Common pitfalls - status = "disabled" : the node exists but the driver won't bind. Change to status = "okay". Most common reason a peripheral doesn't work. - Wrong compatible string : driver won't match the node. Check the driver's MODULE_DEVICE_TABLE or of_match_table for the exact string it expects. One typo = no binding. - Wrong reg address : hardware access fails or wrong peripheral responds. Cross-check with the SoC datasheet. Address cells must match the parent's #address-cells. - Missing parent status : if I2C bus node itself has status = "disabled", enabling a child node does nothing. Enable the bus first. - Wrong interrupt number : device never generates visible interrupts. Interrupt numbering is interrupt-controller-specific; the format is documented in the SoC's DT binding documentation. - Overlay compatible mismatch : overlay's compatible in the root node doesn't match the base DTB. Overlay silently not applied. - I2C address conflict : two devices in the DT at the same address. DT compiles fine but one device fails to probe.

Part 6 : Yocto Integration

Learning objectives - Add custom DTS to a Yocto build - Apply DT patches via bbappend

6.1 Where DTS Files Live in Yocto

For linux-yocto, DTS files live in the kernel source tree at arch/arm64/boot/dts/<vendor>/. The kernel recipe compiles them automatically based on KERNEL_DEVICETREE variable.

# In your machine.conf or local.conf: KERNEL_DEVICETREE = "broadcom/bcm2837-rpi-3-b.dtb" # For multiple DTBs: KERNEL_DEVICETREE = "broadcom/bcm2837-rpi-3-b.dtb \ broadcom/bcm2837-rpi-3-b-plus.dtb" # Verify which DTBs were built: $ ls tmp/deploy/images/raspberrypi3/*.dtb

6.2 Adding a Custom DTS via bbappend

Worked example: custom DTS for a product carrier board # meta-allegion/recipes-kernel/linux/linux-yocto_%.bbappend FILESEXTRAPATHS:prepend := "${THISDIR}/files:" SRC_URI += "file://allegion-controller.dts \ file://0001-add-allegion-board.patch" # Add our board's DTB to the list KERNEL_DEVICETREE:append = " allegion/allegion-controller.dtb" do_configure:append() { # Copy our DTS into the kernel source tree before compilation cp ${WORKDIR}/allegion-controller.dts \ ${S}/arch/arm64/boot/dts/allegion/ }. The DTS file is placed in your layer's files/ directory. The bbappend copies it into the kernel source tree at configure time and adds it to KERNEL_DEVICETREE so the build system knows to compile it.

6.3 Adding DT Overlays via Yocto

# In local.conf or machine.conf (for meta-raspberrypi): RPI_EXTRA_CONFIG = "dtoverlay=bmp280" # Or install the .dtbo file via a recipe: do_install:append() { install -d ${D}/boot/overlays install -m 0644 ${WORKDIR}/bmp280.dtbo ${D}/boot/overlays/ }

Try this yourself - Download any DTS file from the Linux kernel source (e.g., arch/arm/boot/dts/am335x-boneblack.dts). Compile it with dtc. Decompile it back. Spot differences. - Boot a Pi or BBB with a known-working image. Explore /proc/device-tree/ to find the UART and I2C nodes. Check their status property. - Write a minimal overlay to add a fictional I2C device at address 0x50. Compile it to .dtbo with dtc. - In a Yocto build, set KERNEL_DEVICETREE and verify the .dtb file appears in tmp/deploy/images/. - Add of_devmgmt=debug to your kernel command line. Boot and look for DT-related messages in dmesg.

References

- Device Tree specification: devicetree.org/specifications/

- Linux kernel DT documentation: kernel.org/doc/html/latest/devicetree/

- DT binding docs: kernel.org/doc/html/latest/devicetree/bindings/ (one file per driver)

- dtc compiler: git.kernel.org/pub/scm/utils/dtc/dtc.git

- Raspberry Pi overlays: github.com/raspberrypi/linux/tree/rpi-6.1.y/arch/arm/boot/dts/overlays

- Bootlin DT training slides: bootlin.com/doc/training/linux-kernel/