Bootloader and U-Boot

From power-on to kernel handoff: a practical guide

U-Boot (the Universal Bootloader) is the most widely used bootloader in embedded Linux. It bridges the gap between hardware initialization and the kernel, loading the kernel and device tree from storage, configuring memory, and handing control to Linux. This document covers the boot sequence, U-Boot commands and environment, boot scripts, secure boot, and Yocto integration.

How to read this document Parts 1-2 cover the boot sequence and U-Boot fundamentals. Parts 3-4 cover environment variables and boot scripting. Part 5 covers secure boot. Part 6 covers Yocto integration. Keep a board with serial access handy — most of this is best learned interactively.

Prerequisites

- Understand at a high level: power-on, bootloader, kernel, rootfs

- Comfortable on a Linux command line

- Have access to a BeagleBone Black, Raspberry Pi, or QEMU for practice

What you should be able to do after reading

- Explain every stage of the embedded Linux boot sequence

- Navigate the U-Boot prompt and run basic commands

- Read and modify U-Boot environment variables

- Write a boot script (bootcmd) for your product

- Load kernel and DTB from TFTP, SD card, or eMMC

- Integrate a U-Boot recipe into a Yocto build

- Understand the basics of verified/secure boot

Part 1: The Embedded Boot Sequence

Learning objectives - Trace boot from power-on to Linux running - Identify each component's role

1.1 Why Embedded Boot Is Complex

On an x86 PC, the BIOS handles hardware initialization and hands off to GRUB or systemd-boot. The platform is standardized and the bootloader has a rich firmware interface.

On ARM embedded boards:

- Every SoC is different:  different memory maps, different clocks, different initialization sequences

- The CPU starts executing from a fixed ROM address with no memory beyond on-chip SRAM

- External DRAM must be initialized before anything can be loaded into it

- The bootloader has to do real hardware bring-up

1.2 The Generic ARM Boot Sequence

Power on │ ▼ ROM code (hardwired in SoC by manufacturer) • Stack setup, minimal clock config • Scans boot sources (eMMC, SD, UART, USB) • Loads SPL from first bootable source │ ▼ SPL (Secondary Program Loader) — also called MLO on TI, TPL elsewhere • Initializes external DRAM • Initializes storage controller • Loads full U-Boot into DRAM │ ▼ U-Boot • Full hardware initialization • Console, networking, storage drivers • Runs bootcmd (boot script) • Loads kernel image + device tree into DRAM • Passes boot arguments to kernel • Jumps to kernel entry point │ ▼ Linux Kernel • Self-decompresses • Initializes subsystems • Mounts rootfs • Starts systemd (PID 1)

1.3 BeagleBone Black Boot Details

- ROM :TI AM335x ROM code. Checks boot order (eMMC first, then SD). Loads the file named MLO from FAT partition 1.

- SPL/MLO: 128 KB limit (fits in AM335x internal RAM). Initializes DDR3 RAM. Loads u-boot.img into DDR.

- U-Boot: full U-Boot, now in 512 MB DDR. Runs bootcmd which finds and boots the kernel.

- Kernel : loaded from FAT partition 1 (zImage, am335x-boneblack.dtb), rootfs on ext4 partition 2.

1.4 Raspberry Pi Boot Details

RPi is unusual: GPU boots first, ARM CPU is secondary. Covered in depth in the Raspberry Pi with Yocto study guide. U-Boot is optional on RPi, start.elf normally loads the kernel directly. To use U-Boot on RPi:

# In config.txt, redirect boot to U-Boot kernel=u-boot.bin # Then U-Boot loads the actual kernel

Part 2: U-Boot Fundamentals

Learning objectives - Use the U-Boot command prompt - Load files from various sources - Pass information to the kernel

2.1 The U-Boot Prompt

Stop the auto-boot by pressing any key in the countdown window:

Hit any key to stop autoboot: 2 ... 1 ... 0 => # You're now at the U-Boot prompt

2.2 Essential Commands

# Help => help # list all commands => help bootm # help for a specific command # System info => bdinfo # board info (memory map, CPU speed, etc.) => version # U-Boot version # Memory => md 0x82000000 40 # memory display: addr, count (in 4-byte words) => mw 0x82000000 0 10 # memory write: addr, value, count # Storage: MMC (SD/eMMC) => mmc list # list MMC devices => mmc dev 0 # select MMC device 0 (eMMC on BBB) => mmc dev 1 # select MMC device 1 (SD card on BBB) => mmc info # show device info => mmc read 0x82000000 0x800 0x2000 # read: dest_addr, start_blk, count # Storage: FAT filesystem => fatls mmc 1:1 # list files on MMC device 1, partition 1 => fatload mmc 1:1 0x82000000 zImage # load file to memory address # Storage: ext2/ext4 filesystem => ext4ls mmc 1:2 / # list root of ext4 partition => ext4load mmc 1:2 0x82000000 /boot/zImage # Networking => ping 192.168.1.1 # ping host => dhcp # get IP via DHCP => tftp 0x82000000 zImage # load file from TFTP server

2.3 Booting the Kernel

Three boot commands depending on image format:

# bootm: boot legacy image (uImage format) => bootm ${kernel_addr} # bootz: boot zImage (compressed ARM kernel) => bootz ${kernel_addr} - ${fdt_addr} # kernel addr ¦ DTB addr # unused (initrd '-' means none) # booti: boot Image (AArch64 uncompressed kernel) => booti ${kernel_addr} - ${fdt_addr} # Full example: load from SD and boot => fatload mmc 1:1 0x82000000 zImage => fatload mmc 1:1 0x88000000 am335x-boneblack.dtb => setenv bootargs 'console=ttyO0,115200 root=/dev/mmcblk0p2 rw' => bootz 0x82000000 - 0x88000000

Part 3: Environment Variables

Learning objectives - Read and set U-Boot environment variables - Understand bootargs and how they reach the kernel - Save and restore environment

3.1 What the Environment Is

U-Boot stores a set of key-value pairs called the environment. These control everything: where to load the kernel, what arguments to pass, network settings, boot retry behavior. The environment lives in a dedicated region of flash/eMMC and persists across reboots.

3.2 Reading and Setting Variables

# Print all environment variables => printenv # Print one variable => printenv bootcmd bootcmd=run distroboot # Set a variable (in memory only) => setenv myvar 'hello world' # Set a multi-line variable (escape newlines with semicolons) => setenv bootcmd 'fatload mmc 1:1 0x82000000 zImage; bootz 0x82000000' # Save environment to flash/eMMC (persists after reboot) => saveenv # Reset environment to compiled-in defaults => env default -a # Using variable substitution => setenv myload 'fatload mmc ${mmcdev}:${mmcpart} ${loadaddr} ${image}'

3.3 bootargs — The Kernel Command Line

bootargs is the most important environment variable. U-Boot passes its value to the kernel as the command line. The kernel parses it for crucial settings:

# Minimal bootargs setenv bootargs 'console=ttyO0,115200n8 root=/dev/mmcblk0p2 rw rootwait' # console port & baud rootfs device options # Read-only rootfs (production) setenv bootargs 'console=ttyO0,115200 root=/dev/mmcblk0p2 ro rootwait' # With IP (for NFS rootfs development) setenv bootargs 'console=ttyO0,115200 root=/dev/nfs \ nfsroot=192.168.1.100:/nfs/rootfs,v3 \ ip=dhcp rw rootwait' # systemd quiet boot (production) setenv bootargs 'console=ttyS0,115200 root=/dev/mmcblk0p2 ro quiet splash'

3.4 Important Default Variables

- bootcmd — what U-Boot runs automatically after the countdown. The main boot script.

- bootdelay — seconds to wait before auto-executing bootcmd. Set to 0 for production (no delay).

- ipaddr, serverip — network addresses for TFTP boot.

- loadaddr — default address for tftp and similar commands.

- fdtaddr — default address for device tree.

- mmcdev, mmcpart — default MMC device and partition.

Part 4: Boot Scripts

Learning objectives - Write a production bootcmd - Handle boot failure gracefully - Use distro boot

4.1 A Production bootcmd

Worked example: complete boot script for Allegion controller # Set in U-Boot environment or compiled-in defaults setenv kernel_addr 0x82000000 setenv fdt_addr 0x88000000 setenv mmcdev 0 setenv mmcpart 1 setenv bootargs 'console=ttyO0,115200n8 root=/dev/mmcblk0p2 ro \ rootwait quiet splash' setenv load_kernel \ 'fatload mmc ${mmcdev}:${mmcpart} ${kernel_addr} zImage' setenv load_fdt \ 'fatload mmc ${mmcdev}:${mmcpart} ${fdt_addr} am335x-boneblack.dtb' setenv try_boot \ 'run load_kernel; run load_fdt; bootz ${kernel_addr} - ${fdt_addr}' # Main boot command: try to boot, fall back to recovery if it fails setenv bootcmd \ 'if run try_boot; then; else run recovery_boot; fi' # No delay in production setenv bootdelay 0 saveenv. The if run try_boot pattern is key: if the normal boot fails (load fails, kernel panics), U-Boot can fall back to a recovery partition. This pairs with A/B partition schemes.

4.2 distro_bootcmd : The Modern Standard

Modern U-Boot includes distro_bootcmd, which probes multiple boot sources in order (USB, SD, eMMC, PXE, DHCP) using a standardized boot script. It's the default bootcmd on many boards and works with the /boot/extlinux/extlinux.conf convention.

# /boot/extlinux/extlinux.conf (on the rootfs) label Linux Default kernel /zImage fdt /am335x-boneblack.dtb append root=/dev/mmcblk0p1 ro rootwait console=ttyO0,115200 # U-Boot finds this automatically and uses it

4.3 TFTP Boot for Development

# On your build host, set up TFTP server: $ sudo apt-get install tftpd-hpa $ sudo cp zImage /var/lib/tftpboot/ $ sudo cp am335x-boneblack.dtb /var/lib/tftpboot/ # At U-Boot prompt on the board: => setenv serverip 192.168.1.100 # your build host IP => setenv ipaddr 192.168.1.10 # board's IP => tftp 0x82000000 zImage => tftp 0x88000000 am335x-boneblack.dtb => setenv bootargs 'console=ttyO0,115200 root=/dev/mmcblk0p2 rw rootwait' => bootz 0x82000000 - 0x88000000 # Automate with a one-liner: => setenv netboot 'dhcp; tftp 0x82000000 zImage; tftp 0x88000000 bbb.dtb; \ setenv bootargs ...; bootz 0x82000000 - 0x88000000'

Part 5: Secure Boot Basics

Learning objectives - Understand the verified boot chain - Know what U-Boot FIT images provide - Plan for secure boot in a product

5.1 The Problem: Unsigned Boot

Without verified boot, an attacker who can write to the SD card or eMMC can replace the kernel or bootloader with malicious code. The device boots it without question. For security products like Allegion's access controllers, this is an unacceptable attack vector.

5.2 The FIT Image: Signed Kernel + DTB

FIT (Flattened Image Tree) is a U-Boot image format that bundles kernel, DTB, and optional initramfs into a single file, with cryptographic signatures over each component.

# FIT image source (.its file) /dts-v1/; / { description = "Allegion Controller Boot Image"; images { kernel@1 { data = /incbin/("zImage"); type = "kernel"; arch = "arm"; os = "linux"; compression = "none"; load = <0x82000000>; entry = <0x82000000>; hash@1 { algo = "sha256"; }; }; fdt@1 { data = /incbin/("am335x-boneblack.dtb"); type = "flat_dt"; arch = "arm"; compression = "none"; hash@1 { algo = "sha256"; }; }; }; configurations { default = "conf@1"; conf@1 { kernel = "kernel@1"; fdt = "fdt@1"; signature@1 { algo = "sha256,rsa2048"; key-name-hint = "allegion-boot-key"; sign-images = "fdt", "kernel"; }; }; }; };

# Build and sign the FIT image $ mkimage -f boot.its boot.itb $ mkimage -F -k /path/to/keys -K u-boot.dtb -r boot.itb # key directory embed pubkey in U-Boot DTB

5.3 The Verified Boot Chain

- Root of trust: U-Boot compiled with the public key baked in. Attacker can't replace U-Boot without physical access to reprogram flash.

- U-Boot verifies FIT: before booting, U-Boot checks the FIT image signature against its embedded public key. Unsigned or incorrectly signed images are rejected.

- Kernel verifies rootfs: via dm-verity (cryptographic hash tree over the block device). Covered in the A/B Updates and RO Root guide.

- Full chain: hardware (trust anchor) → SPL → U-Boot → FIT image (kernel+DTB) → dm-verity rootfs

Part 6: Yocto Integration

Learning objectives - Add a U-Boot recipe to a Yocto build - Configure U-Boot via bbappend - Use WIC for partition layout

6.1 The U-Boot Recipe

# Check available U-Boot recipes in your layers $ bitbake-layers show-recipes ¦ grep u-boot # Common recipes: # u-boot (from meta-openembedded) # u-boot-fslc (NXP/Freescale) # u-boot-ti-staging (TI boards via meta-ti) # u-boot-raspberrypi (RPi via meta-raspberrypi) # Build U-Boot alone $ bitbake u-boot # Output appears in: $ ls tmp/deploy/images/<machine>/ u-boot.img MLO u-boot.bin

6.2 Configuring U-Boot via bbappend

Worked example: custom U-Boot config for BBB # meta-allegion/recipes-bsp/u-boot/u-boot_%.bbappend FILESEXTRAPATHS:prepend := "${THISDIR}/files:" # Override the defconfig UBOOT_MACHINE = "am335x_boneblack_vboot_defconfig" # Add config fragments SRC_URI += "file://allegion-uboot.cfg" # Add custom environment SRC_URI += "file://0001-allegion-default-env.patch". The UBOOT_MACHINE variable selects which U-Boot defconfig to use (matches files in configs/ in the U-Boot source). The bbappend adds your product-specific modifications on top.

6.3 WIC and Partition Layout

Yocto's wic tool creates the final SD card or eMMC image with the correct partition table. The layout is defined in a .wks file:

# meta-allegion/wic/allegion-bbb.wks # Boot partition: FAT32 with MLO, u-boot.img, kernel, DTB part /boot --source bootimg-partition --ondisk mmcblk \ --fstype=vfat --label boot --active --align 4 --size 40M # Rootfs partition: ext4 part / --source rootfs --ondisk mmcblk \ --fstype=ext4 --label root --align 4 # In local.conf or image recipe: # WKS_FILE = "allegion-bbb.wks" # IMAGE_FSTYPES += "wic wic.bz2"

Common pitfalls - bootdelay = 3 in production. The default is 3 seconds. An attacker with console access can stop at U-Boot and boot anything. Set bootdelay=0 and disable the console in production. - saveenv on a development board. Saving a development environment (with TFTP settings, debug flags) to eMMC and shipping the board with that environment. Always start from a production environment. - SPL filename must be MLO on TI boards. The ROM looks for this exact filename on FAT partition 1. Any other name and the board doesn't boot. Silent failure. - DRAM initialization order. SPL initializes DRAM; if SPL is wrong (wrong DDR timing), U-Boot loads but crashes immediately. Check the hardware reference manual timing parameters. - U-Boot environment corruption. If power is lost while saveenv is running, the environment region in flash can be partially written and corrupted. Use redundant environment (CONFIG_ENV_REDUNDANT=y) for production. - FIT signing keys in the build tree. Never commit private signing keys to your git repository. Store them in a separate, access-controlled key store. The build fetches the public key; signing happens in a separate step.

Try this yourself - Boot a BeagleBone Black or Raspberry Pi. Press a key to stop at U-Boot. Run printenv. Identify bootcmd, bootargs, bootdelay. - At the U-Boot prompt, run mmc list and mmc info. Load the kernel to memory with fatload. Check it loaded with md 0x82000000 40. - Change bootdelay to 5, run saveenv. Reboot. Verify you have 5 seconds. Then set it back to 2 and saveenv again. - Write a custom bootcmd that loads kernel from MMC and prints 'Allegion boot' before booting. Set it, saveenv, reboot. - In Yocto, set UBOOT_MACHINE to the defconfig for your board. Build with bitbake u-boot. Verify MLO and u-boot.img appear in deploy.

References

- U-Boot documentation: u-boot.readthedocs.io

- U-Boot source: source.denx.de/u-boot/u-boot.git

- Bootlin bootloader training: bootlin.com/training/

- FIT image format: u-boot.readthedocs.io/en/latest/usage/fit/index.html

- Yocto U-Boot integration: docs.yoctoproject.org/bsp-guide/

- AM335x Technical Reference Manual (for BBB register addresses)