SDK and devtool

Developing applications against a Yocto SDK

The Yocto SDK provides a cross-compilation environment that application developers can use without needing the full Yocto build system. devtool is the modern recipe-development workflow tool. Together they separate platform engineering (building the OS image) from application engineering (writing code that runs on it).

How to read Parts 1-2 cover the SDK: what it is, how to generate and install it. Part 3 covers using it to compile applications. Part 4 covers the extensible SDK (eSDK) and devtool. Part 5 covers the devtool workflow for recipe development.

What you should be able to do after reading

- Generate a Yocto SDK for your target hardware

- Install and use the SDK to cross-compile a simple program

- Use the SDK with CMake and autotools projects

- Use devtool add, devtool modify, devtool finish

- Iterate on recipe development without full rebuilds

Part 1 — The Standard SDK

Learning objectives - Generate and install an SDK - Understand what the SDK contains

1.1 What the SDK Is

The SDK is a self-contained tarball that installs a cross-compilation environment for your target. It contains:

- Cross-compiler — gcc/g++ configured to produce binaries for your target architecture

- Target sysroot — headers, libraries, and pkg-config files for everything in your image

- Native sysroot — host tools needed during build (cmake, pkg-config, etc.)

- Environment setup script — configures CC, CXX, CFLAGS, LDFLAGS, PKG_CONFIG_PATH for your target

1.2 Generating the SDK

# Generate SDK for your image $ bitbake core-image-minimal -c populate_sdk # For extended SDK with devtool support: $ bitbake core-image-minimal -c populate_sdk_ext # Output: $ ls tmp/deploy/sdk/ poky-glibc-x86_64-core-image-minimal-cortexa57-raspberrypi3-toolchain-4.0.sh # The filename tells you: # host arch (x86_64) → target image → target arch → machine → version

1.3 Installing the SDK

# Install (default to /opt/poky/<version>) $ chmod +x poky-glibc-x86_64-*.sh $ ./poky-glibc-x86_64-*.sh # Press Enter to accept default install location # Or specify location: $ ./poky-glibc-x86_64-*.sh -d ~/sdk-raspberrypi3 # What got installed: $ ls /opt/poky/4.0/ environment-setup-cortexa57-poky-linux # source this to activate sysroots/ cortexa57-poky-linux/ # target sysroot x86_64-pokysdk-linux/ # native tools

Part 2 — Using the SDK

Learning objectives - Activate the SDK environment - Cross-compile a program with and without build system

2.1 Activating the SDK

# Source the environment setup script in every terminal you use $ source /opt/poky/4.0/environment-setup-cortexa57-poky-linux # Verify: CC and CXX now point to the cross-compiler $ echo $CC aarch64-poky-linux-gcc --sysroot=/opt/poky/4.0/sysroots/cortexa57-poky-linux $ echo $CFLAGS -O2 -pipe -g -feliminate-unused-debug-types -fcanon-prefix-map ... $ $CC --version aarch64-poky-linux-gcc (GCC) 11.3.0

2.2 Compiling a Simple Program

# After sourcing the environment: $ $CC -o hello hello.c # Verify it's an ARM64 binary (not x86): $ file hello hello: ELF 64-bit LSB executable, ARM aarch64 # Copy to target and run: $ scp hello root@raspberrypi3:/tmp/ # On target: $ /tmp/hello Hello, world!

2.3 CMake Project with SDK

# After sourcing the SDK environment, CMake just works: $ mkdir build && cd build $ cmake .. \ -DCMAKE_TOOLCHAIN_FILE=$OECORE_NATIVE_SYSROOT/usr/share/cmake/OEToolchainConfig.cmake $ make # CMake picks up CC, CXX, CFLAGS from environment automatically

2.4 Autotools Project with SDK

# Autotools uses CC, CXX, CFLAGS from environment: $ ./configure --host=$HOST # $HOST is set by the SDK env script to the target tuple # e.g. aarch64-poky-linux $ make

Part 3 — The Extensible SDK (eSDK)

3.1 Standard SDK vs eSDK

- Standard SDK — frozen snapshot. Compile against libraries in the image at SDK generation time. Lightweight (~200 MB).

- eSDK — includes devtool and can add/update packages on demand. Heavier (~1-2 GB) but much more powerful for active development.

# Generate eSDK $ bitbake core-image-minimal -c populate_sdk_ext $ ls tmp/deploy/sdk/ poky-glibc-x86_64-core-image-minimal-...-toolchain-ext-4.0.sh # Install eSDK $ ./poky-glibc-x86_64-...-toolchain-ext-4.0.sh

Part 4 — devtool Workflow

Learning objectives - Use devtool add to create a new recipe - Use devtool modify to edit an existing recipe - Use devtool finish to push back to a layer

4.1 devtool add — Creating a New Recipe

Worked example: devtool add for a git project # Source the build environment first $ source poky/oe-init-build-env # devtool add: create recipe and set up workspace $ devtool add myapp https://github.com/example/myapp.git # devtool: # 1. Clones the repo into workspace/sources/myapp/ # 2. Inspects source (detects build system) # 3. Generates workspace/recipes/myapp/myapp_git.bb # 4. Adds workspace layer to bblayers.conf # Try to build it $ devtool build myapp # Edit the recipe $ devtool edit-recipe myapp # Deploy to a running target (for rapid testing) $ devtool deploy-target myapp root@192.168.1.100 # When happy, finish: move recipe to your permanent layer $ devtool finish myapp meta-allegion # devtool: # 1. Moves recipe to meta-allegion # 2. Generates patch files for any source changes # 3. Cleans up workspace.

4.2 devtool modify — Editing an Existing Recipe

# Set up a workspace to modify an existing recipe $ devtool modify openssh # Source code is now in workspace/sources/openssh/ # It's a git repo; make your changes and commit them $ cd workspace/sources/openssh/ $ vim auth.c # make your change $ git add auth.c $ git commit -m 'Allegion: add custom audit hook' # Rebuild with your change $ devtool build openssh # Deploy to target for testing $ devtool deploy-target openssh root@192.168.1.100 # Finalize: generate .bbappend with patches $ devtool finish openssh meta-allegion

4.3 devtool update-recipe

# Update a recipe to a new upstream version $ devtool upgrade myapp --version 2.0 # devtool fetches 2.0, tries to apply existing patches, # reports any that no longer apply cleanly. # Edit workspace to resolve, then finish.

Common pitfalls - Changes in devshell don't make patches automatically. You must git commit your changes inside the workspace source directory before devtool finish. Un-committed changes are lost. - devtool finish overwrites existing recipe. If meta-allegion already has a recipe for this package, devtool finish replaces it. Double-check before running. - eSDK requires disk space. The eSDK is 1-2 GB installed. Make sure application developers have enough disk before distributing it. - SDK environment must be re-sourced in every shell. The source environment-setup-* script sets environment variables for that terminal session only.

Try this yourself - Generate an SDK for core-image-minimal. Install it. Source the environment. Verify CC points to an aarch64 (or ARM) compiler. - Cross-compile a hello world C program using the SDK. Verify it's an ARM binary with file. Copy to a Pi and run it. - Use devtool add to create a recipe for a small open-source project. Build it. Inspect the generated .bb. - Use devtool modify on an existing recipe (try htop or nano). Change one line of source. Build. Deploy to target.

References

- Yocto Project SDK manual: docs.yoctoproject.org/sdk-manual/

- devtool documentation: docs.yoctoproject.org/ref-manual/devtool-reference.html

- eSDK: docs.yoctoproject.org/sdk-manual/extensible.html