Raspberry Pi with Yocto

Building production embedded Linux on Raspberry Pi

This document is a focused practical guide for building Yocto images for Raspberry Pi hardware. It complements the broader Yocto study guides by going deeper on RPi-specific topics: the BSP layer, the three RPi-specific images, the GPU-first boot process, config.txt configuration, all the Pi variants (Pi Zero through Pi 4 and Compute Modules), SSH access via dropbear, and production considerations specific to Pi hardware.

Reading time: roughly 60-75 minutes. The first build will take 1-3 hours; have a Pi and SD card ready if you want to follow along practically.

How to read this document Each part is largely standalone. If you’re just starting, read straight through. If you’re experienced with Yocto but new to RPi, jump to Part 5 (boot process) and Part 6 (configuration). Same callout colors as previous guides: pink for apply-at-work, blue for worked examples, green for practice, amber for pitfalls.

Prerequisites

- Comfortable with the previous Yocto study guides (or equivalent experience)

- A 64-bit Linux workstation with Poky cloned

- A Raspberry Pi (any model from Zero through Pi 4) and a microSD card (8 GB+)

- A USB-TTL serial adapter (e.g., PL2303) for console access

- A 5V power supply (2A+ for Pi 3, 3A for Pi 4)

What you should be able to do after reading

- Choose the right Raspberry Pi model for an embedded product

- Build a working Yocto image for any Pi model

- Choose between rpi-hwup-image, rpi-basic-image, and rpi-test-image

- Explain the GPU-first boot process and what each stage does

- Configure UART, GPU memory split, HDMI, and WiFi via local.conf and config.txt

- Set up SSH access with dropbear or openssh

- Avoid the common RPi-Yocto pitfalls (power, SD cards, UART/Bluetooth conflict)

- Decide whether RPi is appropriate for your production product

Part 1 — Why Raspberry Pi for Yocto

Learning objectives - Decide when Yocto on Pi makes sense vs Raspbian/RPi OS - Distinguish hobbyist Pi use from product Pi use

1.1 Yocto vs Raspbian/RPi OS

Raspberry Pi Foundation’s official OS (Raspberry Pi OS, formerly Raspbian) is a Debian-based distribution. It’s excellent for hobbyist use, education, and prototyping. It is not what you ship in a product.

Use Yocto on Pi when you need:

- Custom packages only — exact package set, no unnecessary services or attack surface

- Reproducible builds — the same image, built today and three years from now

- Production-grade init and update — systemd-based, OTA-friendly, signed updates

- Read-only root filesystem — critical for flash longevity and crash recovery

- Custom kernel configuration — only the drivers you need, security hardening

- Long-term maintainability — your build is in git; rebuild from any commit at any time

- Cross-Pi-model support — one source tree builds for Pi Zero through Pi 4

Use Raspbian when:

- Prototyping or proof-of-concept

- Education or classroom

- Personal projects with no production requirement

- You need rapid iteration and don’t care about build reproducibility

1.2 When RPi Makes Sense for a Product

- Single-board solution with Linux, GPIO, WiFi, Bluetooth, video, audio all on-board

- Volume small enough that Pi pricing is acceptable (~$25-75/unit at retail)

- Product is not safety-critical (Pi is not certified for industrial/medical use without custom validation)

- Operating temperature range is moderate (commercial 0°C to 50°C, not industrial -40°C to +85°C)

1.3 When RPi Doesn’t Make Sense

- Industrial environments — use Compute Module 4 (industrial version available) or move to NXP i.MX, TI Sitara, Rockchip

- Real-time requirements — Pi has no hard-real-time guarantees; consider STM32 + Linux SoC combo

- Battery-powered — Pi draws too much power; ESP32 or similar microcontroller better

- Cost-sensitive at volume — below $5 BOM is impossible with Pi; consider AllWinner, Rockchip

- Long-term parts availability requirements — Raspberry Pi Foundation has supply chain history of shortages; industrial customers prefer dedicated industrial SoCs

The Compute Module reality For production designs that need RPi’s software ecosystem but a smaller/integrated form factor, the Compute Module variants (CM3, CM4) are designed for embedded products. They come on a SODIMM-style edge connector or pads. You design a custom carrier board around them. Yocto support exists via the same meta-raspberrypi layer. This is how serious RPi-based products usually ship.

Part 2 — meta-raspberrypi: The BSP Layer

Learning objectives - Get meta-raspberrypi and its dependencies - Match branches across layers - List supported Pi models

2.1 What meta-raspberrypi Provides

meta-raspberrypi is the hardware-specific BSP overlay for Raspberry Pi. It contains:

- Machine configurations for every Pi model (conf/machine/raspberrypi*.conf)

- Kernel recipes with Pi-specific patches and configuration

- Bootloader files (bootcode.bin, start.elf, fixup.dat — Broadcom’s closed-source firmware)

- Three image recipes specific to Pi use cases (Part 4)

- Device tree blob recipes for each Pi model

- Pi-specific packages (raspi-config helpers, omxplayer for video, etc.)

Source: git://git.yoctoproject.org/meta-raspberrypi (or GitHub mirror at github.com/agherzan/meta-raspberrypi).

2.2 Supported Hardware

Every Pi variant since the original is supported:

Raspberry Pi (original) → MACHINE = "raspberrypi" Raspberry Pi 2 → MACHINE = "raspberrypi2" Raspberry Pi 3 (32-bit) → MACHINE = "raspberrypi3" Raspberry Pi 3 (64-bit) → MACHINE = "raspberrypi3-64" Raspberry Pi 4 (32-bit) → MACHINE = "raspberrypi4" Raspberry Pi 4 (64-bit) → MACHINE = "raspberrypi4-64" Raspberry Pi Zero → MACHINE = "raspberrypi0" Raspberry Pi Zero W (wireless) → MACHINE = "raspberrypi0-wifi" Raspberry Pi Zero 2 W → MACHINE = "raspberrypi0-2w-64" Compute Module 3 → MACHINE = "raspberrypi-cm3" Compute Module 4 → MACHINE = "raspberrypi-cm4"

2.3 Layer Dependencies

meta-raspberrypi depends on meta-openembedded sub-layers for additional packages. Specifically:

- meta-oe — general-purpose extra packages

- meta-multimedia — multimedia packages (gstreamer, alsa-utils, omxplayer)

- meta-networking — networking tools

- meta-python — Python modules

Without these, BitBake will fail with “Nothing PROVIDES X” errors during parsing.

2.4 Getting the Right Branch

Critical rule: every layer must be on the SAME branch as Poky. Mixing branches causes silent failures or build errors.

$ cd ~/yocto # Clone the layers $ git clone git://git.yoctoproject.org/poky $ git clone git://git.openembedded.org/meta-openembedded $ git clone git://git.yoctoproject.org/meta-raspberrypi # Check out the SAME branch on each (scarthgap is current LTS in 2026) $ cd poky && git checkout scarthgap && cd .. $ cd meta-openembedded && git checkout scarthgap && cd .. $ cd meta-raspberrypi && git checkout scarthgap && cd .. # Verify $ for d in poky meta-openembedded meta-raspberrypi; do echo -n "$d: "; cd $d && git rev-parse --abbrev-ref HEAD; cd ..; done

Part 3 — First Build for Raspberry Pi 3

Learning objectives - Configure layers, MACHINE, and local.conf for RPi - Build your first RPi image end-to-end - Locate the build outputs

3.1 Setting Up the Build

$ cd ~/yocto $ source poky/oe-init-build-env build_pi3 # (you are now in build_pi3/; conf/local.conf and conf/bblayers.conf are created) # Add the required layers (order matters: dependencies first) $ bitbake-layers add-layer ../meta-openembedded/meta-oe $ bitbake-layers add-layer ../meta-openembedded/meta-multimedia $ bitbake-layers add-layer ../meta-openembedded/meta-networking $ bitbake-layers add-layer ../meta-openembedded/meta-python $ bitbake-layers add-layer ../meta-raspberrypi # Verify $ bitbake-layers show-layers # Set MACHINE $ echo 'MACHINE = "raspberrypi3"' >> conf/local.conf # Optional: save disk space $ echo 'INHERIT += "rm_work"' >> conf/local.conf $ LICENSE_FLAGS_ACCEPTED += “synaptics-killswitch”

3.2 Trigger the Build

# Build the simplest RPi image first $ bitbake rpi-test-image # First build: 1-3 hours depending on hardware # Subsequent builds: minutes (sstate cache helps)

3.3 Where Outputs Go

$ ls tmp/deploy/images/raspberrypi3/ bcm2710-rpi-3-b.dtb # device tree bcm2710-rpi-3-b-plus.dtb bcm2710-rpi-cm3.dtb bootfiles/ # GPU firmware + config ├── bootcode.bin # GPU first-stage ├── start.elf # GPU firmware ├── fixup.dat # memory config └── config.txt # hardware config kernel7.img # Linux kernel rpi-hwup-image-raspberrypi3.rpi-sdimg # full SD card image ← USE THIS rpi-hwup-image-raspberrypi3.tar.bz2 # rootfs tarball rpi-hwup-image-raspberrypi3.manifest # package list ...

The .rpi-sdimg file is the complete SD card image — partition table, FAT boot partition with GPU firmware, ext4 root with Linux. Flash this directly with dd.

Part 4 — The Three RPi-Specific Images

Learning objectives - Choose between rpi-hwup-image, rpi-basic-image, and rpi-test-image - Understand what each image provides - Know when to build a custom image instead

4.1 The Image Hierarchy

meta-raspberrypi provides three image recipes. Each builds on the previous — they form a layered hierarchy:

rpi-hwup-image ← minimum: based on core-image-minimal ↓ inherits from rpi-basic-image ← adds: splash screen ↓ inherits from rpi-test-image ← adds: meta-raspberrypi packages (multimedia, tests)

# Find them $ ls ../meta-raspberrypi/recipes-*/images/ rpi-basic-image.bb rpi-hwup-image.bb rpi-test-image.bb # Inspect what each contains $ cat ../meta-raspberrypi/recipes-core/images/rpi-hwup-image.bb $ cat ../meta-raspberrypi/recipes-core/images/rpi-basic-image.bb $ cat ../meta-raspberrypi/recipes-core/images/rpi-test-image.bb

4.2 rpi-hwup-image — Hardware Up

“Hardware up” means “boot the hardware to a console.” The minimum needed to verify the kernel boots and the rootfs is functional.

- Based on: core-image-minimal

- Includes: kernel modules, busybox, basic shell

- Excludes: SSH, splash screen, networking tools, multimedia

- Size: ~50 MB

- Use case: smoke-testing a new Pi setup, bring-up of a custom carrier board, baseline for measuring your image bloat

4.3 rpi-basic-image — Hardware Up + Splash

- Based on: rpi-hwup-image

- Adds: psplash boot splash screen (IMAGE_FEATURES += "splash")

- Adds: SSH server (dropbear) for remote access

- Size: ~80 MB

- Use case: typical development image; what you SSH into during early development

4.4 rpi-test-image — With meta-raspberrypi Packages

- Based on: rpi-basic-image

- Adds: packages from meta-raspberrypi: bcm2835-tests, omxplayer (hardware video), userland (libraries), gstreamer plugins

- Size: ~200+ MB

- Use case: verifying Pi-specific hardware features (camera, GPU video acceleration, GPIO)

4.5 Which Image When

- First build ever — rpi-hwup-image. Smallest, fastest to build, easiest to debug if something goes wrong.

- Active development — rpi-basic-image. SSH access is essential during development.

- Verifying hardware features — rpi-test-image. When you need to confirm camera or hardware video decode works.

- Production — a CUSTOM image. None of the three are production-ready. You’ll build your own (see Part 9).

Part 5 — RPi Boot Process: GPU First

Learning objectives - Explain why the GPU boots before the CPU - Identify the role of each file on the boot FAT partition - Compare RPi boot to traditional ARM boards (BeagleBone)

5.1 The Unusual Architecture

Raspberry Pi’s boot is unique among ARM boards. The GPU boots first, not the CPU. This is because:

- The Broadcom BCM2837 (and predecessors) was originally designed for set-top boxes

- The VideoCore GPU has its own CPU cores and runs its own OS (VCOS)

- The ARM cores are a secondary subsystem, brought up by the GPU later in boot

- This is the opposite of standard ARM design (where the ARM core boots first and may use a GPU as a peripheral)

5.2 The Four Boot Stages

Stage 1: On-chip ROM (GPU) ├─ CPU is held in RESET ├─ GPU executes a tiny bootloader baked into the SoC ├─ Reads bootcode.bin from SD card's FAT partition └─ Loads bootcode.bin into L2 cache ↓ Stage 2: bootcode.bin (still on GPU) ├─ Enables external SDRAM └─ Loads start.elf into SDRAM ↓ Stage 3: start.elf (VCOS — the GPU's operating system) ├─ Reads config.txt (hardware configuration) ├─ Reads cmdline.txt (Linux kernel command line) ├─ Loads kernel7.img and the .dtb file └─ Finally enables the ARM CPU and starts the kernel ↓ Stage 4: Linux Kernel (ARM CPU) └─ Standard Linux boot: init, services, login

5.3 The Boot FAT Partition Files

These files MUST be on the first partition of the SD card, formatted as FAT:

- bootcode.bin — GPU first-stage bootloader. Loaded by ROM.

- start.elf — Main GPU firmware. “Small,” “normal,” and “x” variants exist (start4.elf for Pi 4).

- fixup.dat — Memory configuration data for start.elf.

- config.txt — Hardware configuration: GPU memory split, HDMI mode, UART enable, audio, GPIO.

- cmdline.txt — Linux kernel command line: console, root device, init.

- kernel7.img (or kernel.img, kernel7l.img, kernel8.img depending on Pi model) — the Linux kernel.

- *.dtb — Device tree blob describing hardware to the kernel.

5.4 Compared to BeagleBone Black (Traditional ARM)

Pi: GPU ROM → bootcode.bin → start.elf → Linux kernel (GPU boots; CPU enabled in stage 3) BBB: ROM → SPL (MLO) → U-Boot → Linux kernel (ARM CPU boots immediately; standard ARM design)

Practical implications:

- Pi boot doesn’t use U-Boot by default (start.elf does U-Boot’s job)

- Pi boot config is in config.txt, NOT in U-Boot environment variables

- The closed-source GPU firmware (bootcode.bin, start.elf) is part of every Pi image — Broadcom proprietary

- You can install U-Boot on Pi if you want chain-loading, signed boot, or environment variables, but it’s opt-in

Part 6 — RPi-Specific Configuration

Learning objectives - Configure UART, GPU memory, HDMI, and audio for your product - Set IMAGE_ROOTFS_EXTRA_SPACE appropriately - Enable or disable WiFi and Bluetooth

6.1 ENABLE_UART — Serial Console

Raspberry Pi 3 does NOT enable the UART by default because that locks the core clock to a fixed frequency, reducing power efficiency. For development you almost always want UART; for production you might disable it for security.

# In conf/local.conf: ENABLE_UART = "1" # What this does: # - Adds 'enable_uart=1' to config.txt # - Sets the core clock to a fixed frequency (so UART baud doesn't drift) # - Makes /dev/ttyAMA0 (Pi 3) or /dev/serial0 the console

6.2 IMAGE_ROOTFS_EXTRA_SPACE

By default, Yocto sizes the rootfs to fit its contents. Production devices often need writable space for logs, application data, or runtime files. Add extra space (in kilobytes):

# In conf/local.conf or your image recipe: # Add 512 MB extra IMAGE_ROOTFS_EXTRA_SPACE = "524288" # Add 1 GB extra IMAGE_ROOTFS_EXTRA_SPACE = "1048576" # Add 4 GB extra IMAGE_ROOTFS_EXTRA_SPACE = "4194304" # Default value: 0

Apply at production: separate data partition For production devices, don’t put writable data on the root filesystem. Better pattern: - Make rootfs read-only (IMAGE_FEATURES += "read-only-rootfs") - Create a separate writable partition for /var/lib/<your-product>/ - Use systemd-tmpfiles to populate /var/log as tmpfs This way, a crash mid-write can’t corrupt the rootfs; only the data partition is at risk. Cleaner separation, easier recovery, fewer hours debugging filesystem corruption at scale.

6.3 GPU Memory Split

The Pi’s 1 GB (Pi 3) or up to 8 GB (Pi 4) of RAM is shared between CPU and GPU. The split is configured at boot time:

# In your image's config.txt (added via bbappend or custom image): # Minimal GPU (headless products, no video): gpu_mem=16 # Default (general use): gpu_mem=128 # Heavy video processing (camera, hardware decode): gpu_mem=256

Headless products should set gpu_mem=16 to maximize CPU-available RAM. If you’re using the camera or hardware video decode, set higher.

6.4 HDMI Configuration

HDMI defaults work for most monitors, but custom resolution or always-on HDMI requires config.txt entries:

# Force HDMI on even if no monitor detected at boot hdmi_force_hotplug=1 # Force a specific group/mode (DMT, CEA) hdmi_group=1 # CEA (TV) hdmi_mode=16 # 1080p 60Hz # Disable HDMI entirely (saves ~25mA) hdmi_blanking=2

6.5 WiFi and Bluetooth

Pi 3 and later have onboard WiFi and Bluetooth. Configure WiFi via wpa_supplicant.conf:

# Create wpa_supplicant.conf in your image: # /etc/wpa_supplicant/wpa_supplicant-wlan0.conf ctrl_interface=DIR=/var/run/wpa_supplicant GROUP=netdev update_config=1 country=IN network={ ssid="YourSSID" psk="YourPassword" key_mgmt=WPA-PSK } # Enable on boot: $ systemctl enable wpa_supplicant@wlan0.service $ systemctl enable systemd-networkd

6.6 Disabling Unused Peripherals (Power Saving)

# In config.txt: # Disable Bluetooth (saves ~10mA + frees UART on Pi 3) dtoverlay=disable-bt # Disable WiFi (saves ~50mA when idle) dtoverlay=disable-wifi # Disable HDMI (saves ~25mA) hdmi_blanking=2 # Disable ACT LED dtparam=act_led_trigger=none dtparam=act_led_activelow=on # Underclock CPU (saves significant power; reduces heat) arm_freq=600 # default 1200 on Pi 3

Part 7 — Building for Other Pi Variants

Learning objectives - Switch between Pi models with just a MACHINE change - Choose 32-bit vs 64-bit kernel - Pick the right Pi for an embedded product

7.1 The Universal Pattern

The same source tree builds for any Pi. Only MACHINE changes:

# Edit conf/local.conf to switch: $ sed -i 's/^MACHINE.*/MACHINE = "raspberrypi4-64"/' conf/local.conf $ bitbake rpi-hwup-image $ sed -i 's/^MACHINE.*/MACHINE = "raspberrypi0-wifi"/' conf/local.conf $ bitbake rpi-hwup-image

7.2 The Models and When to Use Each

Raspberry Pi Zero / Zero W / Zero 2 W

- Specs: 1 GHz single-core (Zero) or 1 GHz quad-core (Zero 2), 512 MB RAM, micro-USB, WiFi (W models)

- MACHINE: raspberrypi0, raspberrypi0-wifi, raspberrypi0-2w-64

- Best for: low-power IoT, sensors, small embedded products where Pi 3 is overkill

- Watch out: 512 MB RAM is tight for full Linux; Zero 2 W is 64-bit ARMv8

Raspberry Pi 3

- Specs: 1.2 GHz quad-core Cortex-A53, 1 GB RAM, WiFi, Bluetooth

- MACHINE: raspberrypi3 (32-bit) or raspberrypi3-64 (64-bit)

- Best for: general-purpose embedded products, prototyping

- Watch out: 64-bit kernel gives better performance for memory-heavy apps but eats more RAM

Raspberry Pi 4

- Specs: 1.5 GHz quad-core Cortex-A72, 2/4/8 GB RAM, gigabit Ethernet, USB 3.0

- MACHINE: raspberrypi4 (32-bit) or raspberrypi4-64 (64-bit)

- Best for: performance-demanding products, applications needing gigabit network, USB 3.0 storage

- Watch out: needs 3A power supply, generates significant heat (heatsink mandatory)

Compute Module 3 / 4

- Specs: same SoC as Pi 3/4 but on a SODIMM-style edge connector (CM3) or LGA pad layout (CM4)

- MACHINE: raspberrypi-cm3, raspberrypi-cm4

- Best for: production embedded products needing custom carrier boards

- Advantages: smaller, customizable I/O, more memory options, industrial temperature range available (CM4)

- Disadvantages: requires custom PCB design

7.3 32-bit vs 64-bit Choice

- 32-bit (raspberrypi3, raspberrypi4) — smaller binaries, less memory per process, mature ecosystem

- 64-bit (raspberrypi3-64, raspberrypi4-64) — better performance for some workloads, required for >4 GB RAM, more modern toolchain

Rule of thumb: for new products, choose 64-bit. The ecosystem has fully matured; performance is generally better; future-proofing is automatic.

Part 8 — Flashing and First Boot

Learning objectives - Flash a Yocto image to an SD card safely - Set up serial console correctly - Verify the Pi boots successfully

8.1 Flashing with dd

$ cd tmp/deploy/images/raspberrypi3 # Identify your SD card device (be careful — wrong device = data loss) $ lsblk # Look for the device that matches your SD card's size (e.g., 8G) # Common: /dev/sdb or /dev/mmcblk0 # Unmount any auto-mounted partitions $ sudo umount /dev/sdb* 2>/dev/null # Flash $ sudo dd if=rpi-hwup-image-raspberrypi3.rpi-sdimg \ of=/dev/sdb \ bs=4M status=progress $ sync # Verify (optional) $ sudo fdisk -l /dev/sdb # Should show: partition 1 (FAT, ~40MB), partition 2 (Linux, rest of card)

8.2 Serial Console Wiring

USB-TTL adapter (PL2303 or similar) connects to the J8 (GPIO) header:

J8 Pin Pi function USB-TTL function ------ ----------- ---------------- 6 GND GND 8 GPIO14 (TX) RX 10 GPIO15 (RX) TX # Note: Pi TX → adapter RX (NOT Pi TX → adapter TX) # Easy to wire backwards; if you see no output, swap RX and TX

8.3 Minicom Setup

# One-time configuration $ sudo minicom -s Serial port setup: Serial Device: /dev/ttyUSB0 Bps/Par/Bits: 115200 8N1 Hardware Flow Control: No Software Flow Control: No Save setup as dfl Exit # Connect $ sudo minicom

8.4 First Boot Output

Insert the SD card, connect serial, apply power. You should see:

[ 0.000000] Booting Linux on physical CPU 0x0 [ 0.000000] Linux version 5.15.x ... [ 0.000000] Machine model: Raspberry Pi 3 Model B Rev 1.2 ... [ 8.234567] systemd[1]: Detected architecture arm. ... Poky (Yocto Project Reference Distro) 4.0 raspberrypi3 /dev/ttyAMA0 raspberrypi3 login: root root@raspberrypi3:~#

Login as root (no password by default with debug-tweaks).

Part 9 — SSH Access on Raspberry Pi

Learning objectives - Add SSH server to your Pi image - Choose between dropbear and OpenSSH - Configure for development vs production

9.1 Dropbear vs OpenSSH

- dropbear — small (~200 KB), minimal features. Default with rpi-basic-image. Good for resource-constrained Pi Zero.

- OpenSSH — full-featured (~5 MB), industry standard. More configuration options, better tooling support.

For Pi 3 and Pi 4, OpenSSH is fine. For Pi Zero with 512 MB RAM, prefer dropbear.

9.2 Enabling SSH

# Method 1: Use rpi-basic-image (dropbear included) $ bitbake rpi-basic-image # Method 2: Add SSH to a minimal image via IMAGE_FEATURES # In conf/local.conf: EXTRA_IMAGE_FEATURES += "ssh-server-dropbear" # or EXTRA_IMAGE_FEATURES += "ssh-server-openssh" # Method 3: Custom image recipe # In your-image.bb: IMAGE_FEATURES += "ssh-server-openssh"

9.3 First SSH Connection

# Find the Pi's IP address # Via serial console: root@raspberrypi3:~# ip addr show wlan0 # Or check your router's DHCP table for the new device # Connect (default: no password for root with debug-tweaks) $ ssh root@192.168.1.100

9.4 Hardening SSH for Production

Default development settings are dangerous for shipping products. For production:

- Remove debug-tweaks from EXTRA_IMAGE_FEATURES (no more passwordless root)

- Add a non-root user with a strong password

- Disable root SSH login: PermitRootLogin no in sshd_config

- Disable password auth, require public keys: PasswordAuthentication no

- Consider on-demand SSH (only listening when authorized; see Network/SSH Hygiene guide)

Reference: the Network Essentials and SSH Hygiene study guide covers this in depth.

Part 10 — Common Raspberry Pi Yocto Pitfalls

Common pitfalls - Underpowered power supply. Pi 3 needs 2.5A minimum, Pi 4 needs 3A. With a phone charger (1A), you’ll see random reboots, kernel panics, SD card corruption. ALWAYS use a proper Pi-rated supply, especially in development. - Cheap/slow SD cards. Class 4 or unbranded cards cause boot failures, slow writes, premature wear-out. Use Class 10 or A1/A2-rated cards from known brands (SanDisk, Samsung). For production, consider eMMC on a Compute Module instead. - Pi 3 UART/Bluetooth conflict. On Pi 3, the primary UART (ttyAMA0) is used by Bluetooth by default. The mini-UART (ttyS0) is what ENABLE_UART=1 gives you, with variable baud rate based on CPU clock. To get the full UART for console, you need dtoverlay=disable-bt in config.txt. Common gotcha; people set ENABLE_UART=1 and wonder why baud is wrong. - Boot order confusion. Pi 3/4 can boot from SD, USB, or network depending on EEPROM config. If your Pi boots into the wrong thing, check the boot order in the bootloader EEPROM. - WiFi country code missing. Without country= in wpa_supplicant.conf, WiFi is disabled per regulatory rules. Pi 3 just won’t connect; symptom is silent failure. - Wrong kernel image type. kernel.img (Pi 1, Pi Zero), kernel7.img (Pi 2, Pi 3 32-bit), kernel7l.img (Pi 4 32-bit), kernel8.img (any 64-bit). Yocto generates the right one based on MACHINE; problems arise if you mix files from different MACHINE builds. - HDMI not working. Pi disables HDMI if no monitor is detected at boot. Set hdmi_force_hotplug=1 in config.txt if you connect a monitor after boot, or for products that may or may not have one. - GPU memory starvation. Default gpu_mem=64 is too low for camera and hardware video decode. Symptoms: camera works briefly then dies; mmal errors. Increase to 128 or 256 if using GPU features. - Heat throttling on Pi 4. Without a heatsink, Pi 4 throttles CPU at 80°C, slowing the system. For sustained loads in enclosed cases, a heatsink + fan is mandatory. - Mismatched layer branches. meta-raspberrypi on kirkstone branch, Poky on scarthgap branch — build errors that look like missing recipes. Always check: cd meta-raspberrypi && git rev-parse --abbrev-ref HEAD. - SD card wear out on writes. Production devices that log heavily to SD will wear the card out in 6-18 months. Use log2ram, separate read-only rootfs, or move logs to an external storage. Better: use a Compute Module with eMMC.

Part 11 — Production Considerations

Learning objectives - Decide whether Pi is appropriate for your product - Plan for long-term parts availability - Choose between regular Pi and Compute Module

11.1 When Pi is OK for Production

- Consumer products at modest volume (~1k-50k/year)

- Indoor, climate-controlled environments

- Network gateway / hub products

- Educational kits, hobbyist-targeted devices

- Products with regular firmware update cycles (Pi’s long-term security maintenance is community-supported)

11.2 When Pi is NOT OK for Production

- Industrial control — no certifications (UL, CE for industrial, IEC 61131)

- Medical devices — no FDA / regulatory pathway

- Automotive — no AEC-Q100 qualification

- Outdoor / extreme temperature — commercial temp range only (Compute Module 4 has industrial variant)

- Real-time — Linux’s preemption is non-deterministic; consider RTOS-based MCU

- High-volume cost-sensitive — Pi is too expensive at scale; AllWinner, Rockchip cheaper

11.3 Long-Term Availability

Pi Foundation’s commitment: production until at least January 2034 for Pi 4 and 2026 for Pi 3. Real-world experience: supply chain disruptions (chip shortages, allocation to priority customers) can leave low-volume customers waiting months. For production, hedge:

- Build inventory of 12 months’ worth at launch

- Have an alternative SoC qualified (NXP, Rockchip)

- Use Compute Module for industrial designs — longer commitment, less consumer focus

11.4 Compute Module 4 for Real Products

If you’re seriously productizing on Pi, look at CM4 not the consumer Pi:

- Industrial temperature range available (-20°C to +85°C)

- Wider memory and storage options (up to 8 GB RAM, up to 32 GB eMMC)

- WiFi and Bluetooth optional (choose with/without per SKU)

- Smaller form factor (55x40 mm)

- Longer Pi Foundation commitment (production until 2031+)

- Custom carrier board designs gives you control over connectors, I/O, power

Reference designs and example carrier boards are available from the Raspberry Pi Foundation.

11.5 Production Image Pattern

Worked example: production Pi image recipe # meta-myproduct/recipes-images/myproduct-prod-image.bb SUMMARY = "Production image for MyProduct on Compute Module 4" LICENSE = "MIT" inherit core-image # Production hardening IMAGE_FEATURES += "read-only-rootfs splash" # NOTE: no debug-tweaks, no tools-debug, no SSH server by default # Our application packages IMAGE_INSTALL = " \ packagegroup-core-boot \ credential-daemon \ motor-control \ config-defaults \ wpa-supplicant \ chrony \ " # 256 MB extra for application data IMAGE_ROOTFS_EXTRA_SPACE = "262144" # Output formats IMAGE_FSTYPES = "wic.bz2 rpi-sdimg". Pair this with a custom config.txt (via bbappend on rpi-config) that: - Disables Bluetooth if not needed (dtoverlay=disable-bt) - Sets gpu_mem=16 for headless products - Enables UART for field service: enable_uart=1 (consider disabling for shipped units) - Sets arm_freq= to underclock if your product doesn’t need full performance (saves power, reduces heat)

Part 12 — Practice and Closing

Try this yourself - Build rpi-hwup-image for MACHINE=raspberrypi3. Flash to SD card. Boot via serial console. Verify login works. - Switch to rpi-basic-image. Rebuild. Flash. Note the splash screen appears at boot. SSH in as root. - Build for raspberrypi3-64 (64-bit). Boot it. Run uname -m to confirm aarch64 architecture. - Add ENABLE_UART = "1" to local.conf. Rebuild rpi-hwup-image. Verify the serial console works without overlay tricks. - Build for raspberrypi0 (Pi Zero, original, 32-bit). Note the smaller binary size of the resulting image. - Create a custom image recipe that extends rpi-basic-image with one additional package (e.g., htop or vim). Verify it appears in the image manifest. - Configure WiFi in your image by including a wpa_supplicant.conf file via SRC_URI in a custom recipe. Boot, verify WiFi connects. - Set IMAGE_FEATURES += "read-only-rootfs" in your image recipe. Rebuild. Boot and verify the rootfs is read-only (mount ¦ grep ' / '). - Build for Compute Module 4 (raspberrypi-cm4) and compare the build output to a regular Pi 4 build. Note any differences in supported peripherals.

References

- meta-raspberrypi documentation — github.com/agherzan/meta-raspberrypi

- Raspberry Pi documentation — www.raspberrypi.com/documentation

- Compute Module 4 datasheet — datasheets.raspberrypi.com/cm4/cm4-datasheet.pdf

- Pi config.txt reference — www.raspberrypi.com/documentation/computers/config_txt.html

- The previous Yocto study guides in this series — ecosystem, recipes, advanced topics

- Network Essentials and SSH Hygiene study guide — for hardening SSH access on Pi-based products

What to Do This Week

One concrete action If you have a Pi and SD card available, complete the full build cycle end-to-end: - Set up the layers (Poky + meta-openembedded + meta-raspberrypi) on the same branch - Source the build environment, configure MACHINE - Build rpi-hwup-image (1-3 hours) - Flash to SD card with dd - Wire up serial console (J8 pins 6, 8, 10) - Boot the Pi and log in - Switch to a different MACHINE (e.g., raspberrypi3-64 if you used raspberrypi3) and rebuild — observe how Yocto reuses sstate cache to speed up the second build This is typically a 4-6 hour exercise (most of it waiting for the first build). After it, you have hands-on experience with every concept in this guide and a working Pi-Yocto setup you can reuse for product development.

Combined with the previous Yocto study guides (ecosystem, recipes, advanced topics) and the operational Linux guides (capabilities, cgroups, systemd, journald, network/SSH), this completes the foundation for shipping Raspberry-Pi-based embedded Linux products with confidence.