Device Tree Debugging Cheat Sheet

When a driver doesn't bind, look here

## Read the live tree

ls /sys/firmware/devicetree/base/ # nodes as directories dtc -I fs -O dts /sys/firmware/devicetree/base ¦ less # decompile to DTS

## What bound, what didn't

ls /sys/bus/platform/devices/ # all platform devices ls /sys/class/ # bound by class (leds, gpio...) cat /sys/kernel/debug/devices_deferred # drivers stuck in EPROBE_DEFER dmesg ¦ grep -iE 'probe¦defer¦of_' # boot binding messages

## GPIO and I2C

cat /sys/kernel/debug/gpio # who owns each GPIO line ls /sys/bus/i2c/devices/ # bound i2c devices i2cdetect -y 1 # scan i2c-1 for addresses

## Five common failures

| Symptom | Likely cause | Check |
| --- | --- | --- |
| Driver never binds | Wrong compatible string | binding doc; the exact string |
| probe failed -EBUSY | GPIO already claimed | /sys/kernel/debug/gpio |
| Child node ignored | Parent bus disabled | parent node status property |
| EPROBE_DEFER forever | Missing clock/regulator | /sys/kernel/debug/devices_deferred |
| Node present, no driver | status = disabled | set status = okay (overlay) |

EPROBE_DEFER is not an error: It means the driver is waiting for a dependency that isn't ready yet. The kernel retries. Look upstream in the DT for the missing clock/regulator/GPIO.