Day 4 Recap Quiz

Instructor copy — with answer key

Q1. [Recall]

List the boot stages in order from power-on to login.

Answer: BootROM (vendor) → SPL (first-stage loader) → U-Boot (full bootloader) → Kernel → initramfs (if used) → systemd → login.

Q2. [MCQ]

Which command shows what took longest during userspace boot?

(a) dmesg  (b) systemd-analyze blame  (c) top  (d) journalctl -b

Answer: (b) systemd-analyze blame.

Q3. [Concept]

A process you deployed runs on your host but segfaults on the target at startup. ldd shows 'not found' for a library. Root cause and fix?

Answer: A missing runtime dependency on the target. Fix by adding the needed package to RDEPENDS (or installing it). This is the build-vs-runtime dependency distinction biting at runtime.

Q4. [Practical]

Write the command to start gdbserver on the target for /usr/bin/myapp, listening on port 2345.

Answer: gdbserver :2345 /usr/bin/myapp

Q5. [Understanding]

Why is an A/B update scheme safer than updating packages in place on a fielded device?

Answer: It's atomic — the update either fully succeeds or rolls back. It's power-fail safe, has a bounded number of end states, and a failed boot reverts to the known-good slot.

Q6. [MCQ]

Which tool gives accurate per-process memory accounting (handling shared pages)?

(a) free  (b) top  (c) smem  (d) df

Answer: (c) smem (reports PSS, which fairly attributes shared pages).

Q7. [Concept]

For an A/B system, what two things must survive a power loss in the middle of an update?

Answer: (1) The currently-active (other) slot must remain bootable until commit. (2) The bootloader must atomically select a slot, with a boot-count fallback that reverts if the new slot fails to boot.

Q8. [Understanding]

Why should products ship on a Yocto LTS release rather than an intermediate release?

Answer: LTS releases get ~5 years of support including CVE/security backports. Intermediate releases get ~7 months — far too short for a multi-year product.

Q9. [Practical]

Write the command to view the kernel command line on a running device.

Answer: cat /proc/cmdline

Q10. [Concept]

Name the standard debugging ladder when something on the device doesn't work.

Answer: dmesg first (kernel / hardware) → journalctl second (services) → strace third (a specific process's syscalls).

## Exploration Challenges (overnight)

Investigate these and be ready to share what you found at tomorrow's kickoff.

- We used coredumpctl. Explore how to pull a core dump off-device and get a backtrace on your host using the SDK's gdb plus the -dbg packages.

- Investigate RAUC vs Mender vs SWUpdate. Pick the one you'd use for your product and justify it in three bullet points.

- Find out what  systemd-analyze critical-chain  shows that  blame  does not.

- Explore a read-only root with an overlay (overlayfs). How would you keep /etc writable while the base rootfs stays read-only?

- Look up hardware watchdog timers and systemd's RuntimeWatchdogSec. How would you make a hung device auto-recover?

- We did not cover remote logging. Find out how to forward journald logs to a central server (systemd-journal-remote or syslog forwarding).