Hands-On Verification Tasks

One per day — proves you can do the day's core skill

## Day 1

Add a second custom package to meta-trainerdemo: a shell script called 'gw-info' that prints the kernel version (uname -r), uptime, and the device's IP address. Build it into core-image-base, boot the image, and run it.

Success: gw-info is installed in /usr/bin on the booted image and runs, printing kernel version, uptime, and IP.

## Day 2

Add a kernel config fragment to meta-trainerdemo that enables a feature currently off (e.g., CONFIG_USB_SERIAL_FTDI_SIO or CONFIG_NTFS_FS). Wire it via a linux-yocto bbappend, rebuild the kernel, boot, and verify.

Success: zcat /proc/config.gz ¦ grep <YOUR_OPTION> on the booted device shows the option enabled.

## Day 3

Generate an SBOM (INHERIT += create-spdx) and a CVE report (INHERIT += cve-check) for your TrainerGW image. Pick one reported CVE, classify it (Patched / Not-applicable / Unpatched), and write a CVE_STATUS justification for it.

Success: An SPDX file and a CVE report both exist for the image, and you have a written CVE_STATUS justification for one chosen CVE.

## Day 4 — Capstone

CAPSTONE: Full lifecycle on TrainerGW. Build the hardened image, flash/boot it (QEMU or hardware), SSH in as a non-root user, deploy a cross-compiled app via the SDK, debug it once with gdbserver, and measure boot time with systemd-analyze.

Success: All steps completed, plus a short written note recording the boot time and one concrete improvement you would make.